Call 47: Technical Assistance for Power Sector Development of South Asian Countries

Report On Institutional Arrangements For Regional Power Market Integration

Prepared for:				Prepared by:

Office of Energy Programs			Deloitte & Touche, LLP

Bureau of Energy Resources		1919 N. Lynn Street

U.S. Department of State			Arlington, VA 22209

Version 1

December, 2024

This work was funded by the U.S. Department of State, Bureau of Energy Resources, Power Sector Program.

This work does not necessarily reflect the views of the United States Government.

DISCLAIMER

This document has been prepared by Deloitte & Touche, LLP (Deloitte) for the U.S. Department of State (State) under a contract between Deloitte and State.  This document does not necessarily reflect the views of the Department of State or the United States Government.  Information provided by State and third parties may have been used in the preparation of this document but was not independently verified by Deloitte.  The document may be provided to third parties for informational purposes only and shall not be relied upon by third parties as a specific professional advice or recommendation.  Neither Deloitte nor its affiliates or related entities shall be responsible for any loss whatsoever sustained by any party who relies on any information included in this document.

# Table of Contents

1.	Introduction	1

1.1	Background - context and need for the study	1

1.2	Our Approach for the study	1

2.	Power Market in the BBINS	2

2.1	Status	2

2.2	Regulatory Landscape	11

2.3	Type of Multilateral Trading Models	13

3.	Minimum requirements for regional power market integration	17

3.1	Political	17

3.2	Technical	19

3.3	Institutional Arrangements	27

4.	Market platforms and products, including derivatives	33

4.1	Regional Power Exchange	33

4.2	Market Products	33

5.	Appendix A: CBET Transmission Line Details	- 1 -

# Acronyms

| Acronym | Definition |
| --- | --- |
| BEH | Bulgarian Energy Holdings |
| BESS | Battery Energy Storage Systems |
| BPA | Blanket Purchase Agreement |
| CI | Confidential Information |
| COR | Contracting Officer Representative |
| DOS | U.S. Department of State |
| ENR | Bureau of Energy Resources |
| ESO | Electricity System Operator |
| EU | European Union |
| GT&D | Generation, Transmission & Distribution |
| KPI | Key Performance Indicator |
| LOE | Level of Effort |
| MW | Megawatt |
| NEK | National Electric Company |
| POP | Period of Performance |
| PSP | Power Sector Program |
| QR | Quarterly Report |
| RE | Renewable Energy |
| RRP | Recovery and Resilience Plan |
| SOW | Scope of Work |
| USG | United States Government |
| VRE | Variable Renewable Energy |
| WP | Work Plan |

# List of Figures and Tables

Figure 1: Generation mix and Installed capacity in Bangladesh	2

Figure 2: Generation Mix and Installed Capacity in Bhutan	3

Figure 3: Generation Mix and Installed Capacity in India	4

Figure 2: Generation Mix and Installed Capacity in Nepal	4

Figure 4: Generation Mix and Installed Capacity for Sri Lanka	5

Figure 5: Maximum Demand vs Demand Met trend in Bangladesh	6

Figure 6: Demand-Supply and Power Export-Import Trend in Bhutan	7

Figure 7: India - Peak Demand and Deficit (FY 2023-24)	7

Figure 8: Peak Demand and Consumption trend	8

Figure 9: Key aspirations/ targets in BBINS region	9

Figure 10: Key elements of Harmonized bilateral model	14

Figure 11: Trade flow in Integrated Power Trading Model	16

Figure 12: Basic considerations for harmonization of wheeling charges	22

Figure 13: Rights and Obligations of involved parties in Third Party Access (TPA)	25

Figure 14: Division of regulatory authority in the United States and the European Union	29

Figure 15: Overview of CCP Function	31

Table 1: Macroeconomic (GDP) trend in Bangladesh	6

# 1Introduction

XXXXXXXX

## Background - context and need for the study

## Our Approach for the study

XXXXXXXXXXX

# Power Market in the BBINS

The BBINS region is witnessing a rapid transformation backed by high economic growth which requires the power sector to support the economic backbone of the region. Most of the countries have planned aggressive supply capacity addition to meet the ever-growing demand for energy. Apart from capacity addition the region is undergoing a transformation journey in terms of the sector portfolio and market regimes. A few of the emerging trends in the region are highlighted in subsequent sections.

## Status

### Energy Mix and Electricity Scenario

The electricity generation mix of the BBINS region plays a crucial role in assessing the potential for cross-border electricity trade. Each country in the region has unique energy resources and needs, creating opportunities for mutually beneficial energy cooperation.

Bangladesh

Bangladesh heavily relies on thermal-based electricity generation, with natural gas (52%), coal (11%), and diesel & furnace oil (23%) collectively contributing 87% of the total generation of 88,450 GWh. The remaining share comes from hydropower, renewables, and imported energy. In terms of installed capacity, gas-based power plants accounted for 43% of the total installed capacity (~24.9 GW) in FY 2022-23. Thus, Bangladesh's heavy reliance on thermal-based electricity generation, particularly natural gas and imported fuels, makes it vulnerable to supply disruptions and price volatility. The limited share of hydropower and renewables in its energy mix further emphasizes the need for diversification to enhance energy security and sustainability. Regional energy cooperation with neighboring countries, particularly leveraging Bhutan and Nepal's abundant hydropower resources and India's diverse energy mix, could help Bangladesh reduce its dependency on fossil fuels, stabilize costs, and meet its growing energy demand more sustainably.

Figure 1: Generation mix and Installed capacity in Bangladesh

Source: BPDB

Bhutan

Bhutan's electricity generation is predominantly hydropower-based, with minimal contributions from solar and wind energy. Out of total generation capacity of 10,700 GWh in 2022, 99.99% is from Hydro while remaining trace amounts of generation is from solar and wind. Accordingly, with respect to installed capacity, hydro accounts for 2334.10 MW, solar for 0.5 MW while wind accounts for 0.16 MW only (as in 2022). Thus, Bhutan’s electricity generation is predominantly reliant on hydropower, which accounts for nearly its entire energy mix. While this provides Bhutan with a clean and sustainable energy source, the country's heavy dependence on hydropower makes it susceptible to seasonal variations and climate-related challenges such as fluctuating rainfall and snowmelt. Regional energy cooperation can enable Bhutan to diversify its electricity trade, optimize surplus generation during peak hydropower seasons, and access additional energy sources during low-output periods.

Figure 2: Generation Mix and Installed Capacity in Bhutan

Source: Ministry of Energy and Natural Resources (MoENR)

India

India, as the largest producer and consumer of electricity in the region, primarily generates power from coal-fired plants, which constitute the largest share of its electricity generation capacity. In addition to coal, India’s power generation mix includes hydro, wind, solar, and nuclear energy, contributing to a more diversified energy portfolio. The growth of renewable energy sources, particularly solar and wind, aligns with the country's efforts to transition towards a greener energy future. India’s advanced transmission infrastructure, coupled with its diverse generation mix, positions the country as a central hub for facilitating regional electricity trade. This strategic advantage allows India to play a pivotal role in enhancing energy security and supporting power exchanges within the broader regional context. Relevant generation and capacity details are provided as follows.

Figure 3: Generation Mix and Installed Capacity in India

Source: CEA, National Power Portal

Nepal

Similar to Bhutan, Nepal is also almost entirely dependent on its hydro based energy generation. Out of total installed capacity of 3.16 GW as on March 2024, 2.99 GW (~95%) is comprised of hydro capacity, while solar, bagasse and thermal make up for remaining 166 MW. In FY 2023-24, entire generation of 2,878 GWh has been completely attributable to hydro based power generation. Therefore, owing to the seasonality aspect associated with hydro power generation, energy cooperation with neighboring countries is and would be crucial for Nepal in meeting its energy needs.

Figure 4: Generation Mix and Installed Capacity in Nepal

Source: NEA

Sri Lanka

Sri Lanka’s electricity generation is primarily reliant on thermal power, including coal and petroleum-based fuels, complemented by hydropower and a growing share of renewables. While hydropower provides a renewable backbone, the country's dependence on imported fuels makes it vulnerable to price fluctuations and supply chain disruptions. Regional energy cooperation could help Sri Lanka diversify its energy sources by accessing surplus electricity from neighboring countries, particularly renewable energy from India and hydropower from Bhutan and Nepal. Such collaboration would enhance energy security, reduce costs, and support Sri Lanka’s transition to a more sustainable energy mix.

Figure 5: Generation Mix and Installed Capacity for Sri Lanka

Source: Ministry of Power and Energy, Sri Lanka

### Demand and Supply

The BBINS region benefits from complementary demand, supply, and load patterns, making substantial case for enhanced energy cooperation. Bhutan and Nepal, with their hydropower-dominated energy mix, generate surplus electricity during the monsoon season, aligning with peak demand periods in energy-deficit countries like Bangladesh and parts of India. Additionally, varying daily load patterns; such as evening peaks in India and Bangladesh, can be balanced through cross-border electricity trade, utilizing surplus generation from Bhutan and Nepal. This complementarity is further enhanced by the region’s diverse generation mix, where renewable hydropower resources in Bhutan and Nepal can offset fossil fuel dependency in Bangladesh and complement the intermittent nature of solar and wind in India. Regional integration would optimize resource utilization, enhance energy security, and support the transition to a sustainable energy future for all BBINS countries.

Bangladesh

Bangladesh had been Asia’s fastest growing economies over the past decade, with average growth of around 6% p.a. Growth had been accelerating, reaching 6.94% in 2021, 7.1% in 2022 and 6.03% in 2023. The accelerated growth in the past decade was attributed to a “demographic dividend”, strong ready-made garment exports and stable macroeconomic conditions. However, recent political unrest in July and August 2024 has affected this growth trend. Factors like reduced export demand, domestic energy shortages and adverse weather constrained agriculture have significantly contributed to decrease in GDP growth rate wherein same is expected to reach 5.8% in 2024 only 5.1% in 2025.

Table 1: Macroeconomic (GDP) trend in Bangladesh

| Macroeconomic parameter | 2015-16 | 2016-17 | 2017-18 | 2018-19 | 2019-20 | 2020-21 | 2021-22 | 2022-23 | 2023-24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GDP growth | 6.56% | 7.11% | 7.28% | 7.86% | 8.20% | 2.37% | 6.94% | 7.1% | 6.03% |

The country’s rising energy demand as witnessed historically, has been driven majorly by industrialization, which makes regional energy cooperation critical for meeting future requirements sustainably. This is substantiated by the fact that historically, electricity peak demand has witnessed a steady growth, with a CAGR of ~9% from 2018-19 to 2022-23 while electricity consumption has grown at a CAGR of 6% during same period. As per the annual report of FY 2022-23 ,peak demand served in 2022/23 was 17,100MW, and total system generation was 88,450 GWh.

Figure 6: Maximum Demand vs Demand Met trend in Bangladesh

*As per the BPDB, the maximum demand served is max. available generation. Source: BPDB

Bhutan

Bhutan has traditionally been a power-surplus nation, with its available energy supply consistently exceeding domestic consumption. This surplus is primarily driven by its hydropower resources, which generate significant electricity during the monsoon season. However, the seasonal nature of hydropower production poses challenges, particularly during the dry winter months when river flows decrease and generation capacity drops. During these periods, Bhutan has relied on power imports from India to meet its domestic demand. This dynamic underscores the need for regional energy cooperation, enabling Bhutan to export surplus electricity during high-generation periods and secure reliable imports during seasonal deficits, ensuring a stable and sustainable energy supply.

Figure 7: Demand-Supply and Power Export-Import Trend in Bhutan

Source: National Statistics Bureau

India

In the past nine years, from FY 2014-15 to FY 2023-24, electricity demand in India has grown at a compound average growth rate of 4.77%, i.e., from 1069 TWh to 1626 TWh. Installation of new generation facilities, strengthening of transmission and distribution networks, demand side management and better management of the existing resources have helped the country meet the rising demand and curtail the unmet demand of energy. Peak electricity demand in India grew at a compound average growth rate of 5.7%, i.e., from ~148 GW in FY 2014-15 to ~243 GW in FY 2023-24.

Each region in India exhibits distinct characteristics in terms of climate and the composition of electricity consumption across residential, commercial, and industrial sectors. These differences result in regional variations in seasonal energy demand. The western and northern regions experience the highest energy demands, with the northern region also recording the country’s most significant energy deficits. Meanwhile, the eastern region, abundant in coal resources, generates substantial energy surpluses and supplies power to other regions.

Overall, India's energy demand peaks during the summer months of April to July, driven by increased cooling requirements. Energy generation remains relatively stable throughout the year because a significant portion comes from non-renewable sources with consistent output. These summer months also record the highest energy deficits. In contrast, energy demand and deficits decrease during the winter months from November to February as cooling needs diminish and heating loads are minimal in India's tropical and subtropical climate. As seen in the graph below, the overall peak demand is higher during May-Oct (summer and festivals) period than Nov-Feb period (winter).

Figure 8: India - Peak Demand and Deficit (FY 2023-24)

Source: Central Electricity Authority

Nepal

The peak power demand in Nepal has increased steadily in the last 5 years (FY20 to FY24) at a CAGR of ~12% to reach 2,212 MW in FY24 from 1,408 MW in FY20. The demand increase has been led by electricity availability, rapid electrification and increasing industrial demand. Similar to Bhutan, being overtly dependent on hydro based power generation, Nepal experiences seasonal variations in electricity supply. During the monsoon, surplus hydropower is available, but in the dry winter months, reduced generation necessitates imports from India.

Figure 9: Peak Demand and Consumption trend

Source: NEA

Sri Lanka

Electricity consumers have grown from 7,022,103 in 2019 to 7,596,894 in 2023. As on 2023 end, the maximum nighttime demand reached 2,414.8 MW, while the peak daytime demand was 2,166 MW. The lowest system demand recorded was 928.4 MW in 2023. Sri Lanka’s reliance on imported fuels for electricity generation and its exposure to fluctuating energy prices emphasize the need for regional energy cooperation. Utilizing surplus hydropower from Bhutan and Nepal or renewable energy from India would enable Sri Lanka to diversify its energy mix, enhance energy security, and reduce its dependence on costly thermal power, supporting a more stable and sustainable energy future.

### Key Aspirations and Targets

The BBINS region is witnessing a rapid transformation backed by high economic growth which requires the power sector to support the economic backbone of the region. Most of the countries have planned aggressive supply capacity addition to meet the ever-growing demand for energy. Apart from capacity addition the region is undergoing a transformation journey in terms of the sector portfolio and market regimes. National targets in the BBINS countries, such as reducing greenhouse gas emissions, increasing the share of renewable energy, and achieving carbon neutrality, create a compelling case for regional energy cooperation. Countries like Bhutan and Nepal, with abundant hydropower resources, can provide low-carbon electricity to neighboring nations, supporting their decarbonization efforts. Similarly, India and Bangladesh, with growing renewable energy capacities in wind and solar, can complement hydropower supply by balancing intermittent generation with regional energy trade. Achieving carbon neutrality requires optimizing resource allocation and leveraging each country’s strengths, such as Bhutan’s surplus hydropower or India’s renewable energy advancements

Figure 10: Key aspirations/ targets in BBINS region

### Existing Trade & Transmission network in the region

CBET Status

Currently, the regional power trade of around 7 GW is bilateral in nature. Until 2023, trade was facilitated through bespoke long-term power purchase agreements.  The table below sets out details of the existing cross-border electricity transactions in the BBIN region.

| Particulars | Source | Agreement Type |
| --- | --- | --- |
| India à Bangladesh (2656 MW) | 250 MW NVVN Ph-I | Government-to-Government |
|  | 200 MW PTC Ph-II | Commercial |
|  | 160 MW Tripura | Government-to-Government |
|  | 300 MW NVVN Bharamara Ph-II | Government-to-Government |
|  | 250 MW Sembcorp | Commercial |
|  | 1496 MW Adani Godda | Commercial |
| Bhutan à India (2380 MW) | 1020 MW Tala | Government-to-Government |
|  | 336 MW Chukha | Government-to-Government |
|  | 60 MW Kurichhu | Government-to-Government |
|  | 126 MW Dagachhu | Commercial |
|  | 720 MW Mangdechhu | Government-to-Government |
|  | 118 MW Nikachhu | Government-to-Government |
| India à Nepal (600-800 MW) | 237 MW India | Government-to-Government |
|  | 400-500 MW Market | Commercial |
| Nepal à India (800-1000 MW) | 941 MW from 28 hydro projects | Commercial |
| Nepal à Bangladesh (40 MW) | 40 MW (through Indian grid) | Government-to-Government (Tripartite Agreement) |

Source: Utility Annual reports, Deloitte research

In 2021, the Central Electricity Authority (CEA) issued the "Procedure for Approval and Facilitating Import/Export (Cross Border) of Electricity by the Designated Authority," enabling power trading through the Indian Energy Exchange (IEX). By October 2021, Nepal began participating in the day-ahead market (DAM) on the IEX, initially trading around 39 MW (24 MW from the Trishuli Hydropower Plant and 15 MW from the Devighat Powerhouse). Over the past three years, Nepal’s trade volume in the DAM has grown significantly, eventually gaining access to the Real-Time Market (RTM). The Nepal Electricity Authority (NEA) has since secured medium-term power sales agreements with distribution companies in Haryana and Bihar. Additionally, India now recognizes hydropower imports from other countries as part of the Hydropower Purchase Obligation (HPO), encouraging Indian buyers to source power from Nepal. In January 2024, the two countries signed an agreement to facilitate 10,000 MW of power exports from Nepal to India within the next decade.

In a landmark development, Nepal, India, and Bangladesh have signed a trilateral agreement for Nepal to export 40 MW of electricity to Bangladesh via India’s grid. This will be Nepal’s first-ever electricity sale to Bangladesh, using the 400 KV Dhalkebar-Muzaffarpur transmission line in India, with export calculations managed at Muzaffarpur by NEA. The Cabinet Committee on Public Purchase (CCPP) in Bangladesh has also approved importing approximately 500 MW from Nepal’s proposed 900 MW Upper Karnali Hydroelectric Project, developed by GMR, which will flow to Bangladesh via NTPC Vidyut Vyapar Nigam Limited through Indian transmission corridors. Additionally, the three countries agreed to collaborate on the Sunkoshi-3 Hydropower Project and to assess the feasibility of a new cross-border transmission line for future energy trade.

In another regional trade development, Bhutan recently began purchasing power from the IEX to meet dry season deficits, importing approximately 1130 GWh in FY 2023-24 to address generation shortfalls.

Transmission Infrastructure

The existing transmission infrastructure between BBINS countries has been instrumental in facilitating cross-border electricity trade, primarily through bilateral arrangements. India serves as the central hub in the region, connecting with Bhutan, Nepal, and Bangladesh through robust transmission networks. Bhutan and India share a well-established grid interconnection that supports the export of Bhutan’s hydropower surplus to India. Similarly, Nepal is connected to India through multiple transmission lines, enabling the exchange of electricity, particularly during Nepal’s seasonal deficits. Bangladesh imports power from India via high-capacity interconnections such as the Baharampur-Bheramara HVDC line. However, Sri Lanka lacks a physical interconnection with the BBIN grid, though proposals for a submarine cable link to India have been explored. Details of existing/ proposed cross-border transmission lines in BBINS region have been provided in Appendix A.

While the existing infrastructure has facilitated growing electricity trade, it remains limited in capacity and coverage to fully support regional integration. Strengthening and expanding transmission networks, including multilateral interconnections, is essential for realizing the potential of a regional power market in BBINS.

## Regulatory Landscape

The current status of the Cross-Border Electricity Trade (CBET) regulatory framework in the BBINS region is characterized by a mix of bilateral agreements and country-specific regulations. India, the largest player in the region, has well-established policies and regulations through the Ministry of Power, Central Electricity Regulatory Commission (CERC), and Central Electricity Authority (CEA). Neighboring countries like Bangladesh, Bhutan, Nepal, and Sri Lanka also have their own regulatory bodies overseeing electricity trade within their borders, but they largely operate in isolation with limited coordination across borders. While some countries, like India and Bangladesh, have initiated agreements for cross-border electricity trade, the regulatory frameworks for CBET remain fragmented, often causing inefficiencies and barriers to broader regional integration.

### Rules/Regulations and Frameworks in Bangladesh

Although, Bangladesh lacks a detailed policy and regulatory framework to promote CBET, the Quick Enhancement of Electricity and Energy Supply (Special Provisions) Act, 2010 played a key role in CBET by allowing the Government and concerned state-owned enterprises to accept appropriate plans for power import to alleviate power shortage in the country. The major legal instruments in Bangladesh for CBET are BERC Act, 2003, The Electricity Act, 2018 and Draft Policy on Cross Border Electricity Trade. The draft addresses key aspects such as grid interconnection, tariffs, system operations, the generation mix for CBET, grid operation recommendations, transmission access, transmission pricing, ancillary services, and dispute resolution mechanisms. It also introduces the concept of an Independent System Operator for Bangladesh, proposing transition from the current single-buyer model in the country's power market.

### Rules/Regulations and Frameworks in Bhutan

The Grid Code policy covers the rules, procedures, guidelines, criteria, and responsibilities to be followed by the operators of the transmission systems. In Bhutan, the grid codes specify same voltage variation limits for both planning and operation stages (+/- 5%). The grid code also categorizes the criteria for classifying an operating state as Normal/Alert/Emergency (+/-5% for normal and +/-10% emergency/alert).

### Rules/Regulations and Frameworks in India

Cross-border electricity trade (CBET) in India is primarily managed by the Ministry of Power in coordination with key power sector entities, including the Central Electricity Regulatory Commission (CERC) and the Central Electricity Authority (CEA). The Ministry of Power oversees policy-level interventions, while CERC and CEA handle regulatory and procedural aspects as the central regulating and planning bodies, respectively. Additionally, power exchange operations and transactions are regulated by CERC through specific regulations, with the National Load Dispatch Centre (NLDC) ensuring grid stability during power flows on the exchange. The Cross Border Electricity Trade of Electricity was commenced in the Day Ahead Market of IEX in 2021-22. The trade with Nepal commenced on 17.04.2021, whereas the trade with Bhutan commenced on 01.01.2022.

### Rules/Regulations and Frameworks in Nepal

Although Nepal does not have a comprehensive policy and regulatory framework specifically designed to promote Cross-Border Electricity Trade (CBET), the primary legislation governing the power sector is "The Electricity Act 1992." This Act includes provisions that facilitate the import and export of electricity. In addition, the "Electricity Regulation 1993," issued under the Act, outlines the application requirements for electricity imports and exports. However, beyond these provisions, the regulatory framework for CBET has not progressed significantly due to the absence of an independent regulatory commission. In 2001, Nepal introduced "The Hydropower Development Policy," which allows the government to grant licenses for the export of electricity from hydropower projects with an installed capacity exceeding 100 MW.

### Rules/Regulations and Frameworks in Sri Lanka

Currently, the Ceylon Electricity Board (CEB) is responsible for generating, transmitting, and distributing electricity through its own network, operating under a single-buyer model. Recently, the government has initiated efforts to unbundle the generation, transmission system operation, and distribution sectors to increase competition and improve service delivery at affordable prices. The goal of this unbundling process is to reduce the burden on the treasury and transform the utility into a financially independent institution. Additionally, the unbundling aims to foster increased investment in renewable energy (RE) development on a competitive basis and promote benchmarking competition among distribution licensees. The Sri Lanka Electricity Act (SLEA) includes provisions for small-scale generation additions under standardized power purchase agreements. The Government of Sri Lanka (GOSL) is  working to integrate into the regional power market, following the completion of ongoing studies. Cross-border interconnections will allow Sri Lanka to benefit economically by importing and exporting electricity during periods of surplus or shortage. This will be especially advantageous for achieving the GOSL's target of 70% renewable energy by 2030. The GOSL will adopt the most suitable business model for this integration, based on the ongoing studies related to grid interconnection with India.

Settlement Nodal Agency (SNA)

NTPC Vidyut Vyapar Nigam Ltd (NVVN) has been designated as the SNA for Bangladesh, Bhutan, Nepal and Myanmar, under the Guidelines for Import /Export (Cross Border) of Electricity 2018, from India’s side. It is therefore responsible for settlement of grid operation related charges like operating charges, charges for deviation. NVVN’s key functions include coordination with Load Dispatch Centers on behalf of Cross Border Entity, provide information on Open Access, undertake weekly meter reading among others. The other billing and payment related activities include raising bill to Cross Border Entity for deviation in scheduling, regional transmission deviation charges, reactive energy charge & congestion charge, system operation fees & charges, Settlement Nodal Agency charges / fees, other statutory and necessary charges and late payment surcharge (if applicable).

### Regional Regulations and Frameworks to Support CBET

There are regional regulations and frameworks in place to support cross-border electricity trade (CBET) which highlight the political commitment of South Asian nations to foster regional energy cooperation. These initiatives aim to strengthen electricity trade by evolving from traditional bilateral arrangements to a more streamlined and efficient regional system. These regional regulations and frameworks are outlined below.

SAARC Framework Agreement

A comprehensive framework was developed under the South Asian Association for Regional Cooperation (SAARC). The “SAARC Framework Agreement for Energy Cooperation (Electricity)” was signed by all member states of SAARC in Nepal in November 2014.

BIMSTEC MoU for Establishment of BIMSTEC Grid Interconnection

The Bay of Bengal Initiative for Multi-Sectoral Technical and Economic Cooperation (BIMSTEC) came into being on 6 June 1997 through the Bangkok Declaration.  The BIMSTEC Energy Centre (BEC) was established on 22 January 2011. A Memorandum of Understanding (MoU) for the Establishment of the BIMSTEC Grid Interconnection was signed during the Fourth BIMSTEC Summit in Kathmandu, Nepal, on 31 August 2018, and came into effect on 7 April 2019. The MoU provides a framework for member countries to collaborate on implementing grid interconnections, facilitating electricity trade, and promoting efficient and optimal power transmission across the BIMSTEC region.

## Type of Multilateral Trading Models

Power trading models vary in complexity and structure, depending on the level of integration and the scope of cooperation between participating countries. In the BBINS region, power trade has primarily been limited to bilateral arrangements. These bilateral trades have served as a good starting point for regional cooperation. However, as the share of variable renewable energy (RE) in the BBINS region increases, along with increasing demand and complementary demand and load profiles, more advanced and structured market setups with shorter time frames may be required. Power trade can range from basic bilateral, unidirectional exchanges to more integrated multilateral trading models, which facilitate trading among multiple jurisdictions. The most advanced models involve unified regional markets, where participants from various states or countries operate within a shared market framework. Currently, the BBINS region primarily operates on bilateral power trading, but there is potential for evolving toward more sophisticated models that enhance efficiency and flexibility in the future.

### Harmonized bi-lateral

A harmonized bilateral power trading model is an approach where neighboring countries or entities engage in cross-border electricity trading through bilateral agreements, while aligning their regulatory frameworks, operational procedures, and grid management practices to ensure smooth and efficient transactions. In this model, a common framework is developed for entering into and managing cross-border bilateral contracts. This framework builds upon existing bilateral arrangements, improving the efficiency of the process and enabling any two interconnected countries to trade with each other, regardless of whether they share a border. The model involves the development of cross-border transmission infrastructure and the coordination of grid operations to ensure stability and reliability of power flows. By providing consistent rules and transparent pricing, the harmonized bilateral model helps optimize power trade while maintaining each country's sovereignty, making it an ideal solution for regions like BBINS, where regional energy cooperation is evolving.

To implement the harmonized bilateral model, three key elements are introduced.

Figure 11: Key elements of Harmonized bilateral model

While harmonized bilateral trading is not equivalent to full multilateral trading, the inclusion of these elements allows for more flexible and diverse bilateral arrangements and lays the foundation for future multilateral trading frameworks.

### Secondary Trading

A secondary trading model serves as a regional power market that market participants can access alongside their domestic systems or markets. In this model, only surplus generation or supply gaps would be traded, with domestic markets clearing first. The secondary market would only be utilized when it offers additional value to the participants. This framework would facilitate comprehensive, multilateral, and multidirectional power trading, while remaining fully compatible with harmonized bilateral trading systems.

To establish this model, several new components must be introduced, some of which build upon the harmonized bilateral model, while others are unique to the secondary approach. For instance, the wheeling model from the harmonized bilateral framework would also be applicable here.

A key feature of the secondary model is the introduction of a regional market operator. This operator would gather data on excess supply and demand, match potential trades, and share relevant information with market participants. It could also assist in calculating wheeling charges. The same institution serving as the regional coordinator in the harmonized bilateral model may take on the role of the market operator.

Given the added complexity of trading in a secondary model, a central clearing party (CCP) would be necessary to manage the financial transactions for cleared trades. The market operator could assume the CCP function, or a separate entity might be established for this purpose.

Case Study of Secondary Trading South African Power Pool (SAPP) was created on 28 August 1995, with the primary aim of providing reliable and economical electricity supply to consumers in each of the SAPP member countries, consistent with reasonable utilization of natural resources and minimized negative impact on the environment. SAPP consists of 12 countries totaling a population of approximately 300 million people. As of 2021, SAPP had 17 members active in cross-border power trade. The main products traded in the SAPP competitive electricity market include the Day-Ahead Market (DAM), Intra-Day Market (IDM), Forward Physical Market – Weekly (FPM-W), and Forward Physical Market – Monthly (FPM-M). The total volume traded across these markets amounted to approximately 1.5 TWh. A key takeaway from the development of SAPP is that national markets have not been fully deregulated. In each participating country, the incumbent power companies operate as single buyers and sellers of electricity. Despite this, a market model has successfully been developed to optimize the regional use of power resources. In some countries, Independent Power Producers (IPPs) are permitted to participate directly in the SAPP. Under the SAPP market framework, only excess generation is traded, meaning member states prioritize covering their domestic demand before offering surplus generation to the regional market. Additionally, power can be traded on an emergency basis to address unforeseen shortages.

### Fully Integrated Trading Model

An integrated primary trading model focuses on regional, multilateral trading as the default option, but it is a long-term goal, which requires development of a secondary trading model as the primary step. This model also requires strong cooperation and alignment of technical, regulatory, and policy frameworks among participants.

In this model, more electricity can be traded based on short-term market and system needs, with the Day-Ahead Market (DAM) being the key focus. This approach can help better use generation resources and transmission infrastructure, particularly as renewable energy becomes a bigger part of the system. However, adopting an integrated primary model would require national market changes to encourage competition and efficient price signals. Opening up markets brings three key benefits: better use of resources, more efficient generation, and wholesale prices that reflect actual market conditions, leading to better consumption patterns.

The integrated primary model would involve trading domestic resources through a regional market operator, optimizing production and consumption based on the region's needs. However, national operators would still be responsible for managing system operations and planning, ensuring that national control remains intact. Typical trade flow example in this model is as follows:

Figure 12: Trade flow in Integrated Power Trading Model

Source: IEA

The primary trading model is a fully integrated market that would stretch across multiple AMS. It will therefore require some new key enabling elements and functions. These are listed as follows:

- Comprehensive national market reforms, including design, organizational adjustments, regulatory frameworks, and legislative changes, to support integrated markets and institutions.

- A capacity-building program aligned with domestic market transformations.

- Streamlining of national procurement processes and market operators, which will be centralized under the regional market operator.

- Incentive-driven measures to enhance liquidity and promote the development of organized pooled auction market segments.

- Option to transition long-term physical contracts into financial contracts, positioning short-term markets (e.g., DAM/IDM) as the primary venue for physical trading.

- Option to adopt a congestion rent model as an alternative to traditional wheeling charges.

Thus, to facilitate transition from a simple bilateral power market to a robust multi-lateral regional framework, same would require developing some key institutional mechanisms/ arrangements which in turn would facilitate fulfilment of essential political and technical requirements for setting up and operating  a regional power exchange successfully.

# 1Minimum requirements for regional power market integration

The successful integration of a regional power market involves meeting a range of minimum requirements across political, technical, and institutional dimensions. Political requirements include establishing cross-border agreements and commitments, while technical requirements focus on grid connectivity, operational protocols, and ensuring reliability in energy exchanges. However, the effective realization of these political and technical prerequisites can only be achieved through the development of robust institutional mechanisms. These institutions play a critical role in coordinating the efforts of various stakeholders, managing market operations, and ensuring compliance with agreed frameworks. Together, these requirements form a comprehensive foundation for a functioning regional power market, where institutional mechanisms act as the key enablers to bring political and technical objectives into action. The major requirements in this regard are accordingly discussed further in subsequent sections.

## Political

The establishment of a regional power exchange in the BBINS region necessitates addressing several political requirements to ensure effective and sustainable cross-border electricity trading. A critical prerequisite would be the development of intergovernmental agreements that establish a robust framework for cooperation among the participating countries. These agreements may include the establishment or designation of specific authorities and regional institutions responsible for overseeing and facilitating the power exchange. Such agreements serve to formalize commitments and delineate roles and responsibilities, providing a stable institutional foundation for integration.

Another essential requirement is achieving consensus on certain non-technical aspects, such as adopting a common working language. A shared linguistic framework will streamline communication and decision-making processes among diverse stakeholders from different jurisdictions, reducing potential misunderstandings and fostering greater collaboration.

A key political factor is the alignment of priorities and commitments among the participating governments. Political support would ensure that regional integration initiatives are accorded adequate attention and resources within national agendas. Cross-border integration requires coordination beyond the technical and institutional domains, emphasizing the importance of sustained governmental engagement and endorsement. Historical examples of successful regional cooperation demonstrate that consistent political backing significantly enhances the likelihood of achieving long-term integration goals.

Therefore, while technical expertise and institutional mechanisms are indispensable, the political dimensions, viz. intergovernmental agreements, common frameworks, and consistent governmental alignment are the foundation upon which a regional power exchange in the BBINS region must be constructed. These elements require sustained dialogue, mutual trust, and a shared vision for regional energy integration to realize the full potential of cross-border electricity trading.

### Intergovernmental agreement(s)

Intergovernmental agreements play a pivotal role in establishing and operationalizing regional power exchanges. These agreements serve as the foundation for political support, enabling collaboration among countries to achieve cross-border electricity trading. By formalizing commitments and creating a framework for cooperation, these agreements set the stage for effective regional energy integration.

At the initial stages, such agreements are crucial for building consensus among participating nations. They provide the legal and institutional framework that allows utilities, regulatory authorities, and other stakeholders to move forward with necessary developments. For instance, they can authorize exemptions or adjustments to existing national laws and regulations to accommodate regional trading mechanisms. This is essential in overcoming policy and regulatory barriers that might otherwise hinder integration efforts.

Additionally, intergovernmental agreements help in defining the level of technical and operational harmonization required for system cooperation. They facilitate the sharing of data, the alignment of market rules, and the coordination of grid operations, which are critical for ensuring the reliability and efficiency of a regional power exchange. Agreements may also establish protocols for dispute resolution, pricing mechanisms, and responsibilities for infrastructure investments.

Examples from successful regional power exchanges, such as the Southern African Power Pool (SAPP) and the Nord Pool in Europe, highlight the importance of these agreements. For SAPP, intergovernmental agreements created a structured environment where member countries collaborated on energy trading while addressing regulatory challenges. Similarly, in the case of Nord Pool, initial agreements among Nordic countries ensured that national utilities could participate in a shared market while adhering to common rules and standards.

Thus, intergovernmental agreements are not merely procedural instruments but are strategic tools for fostering political commitment, enabling regulatory harmonization, and promoting trust among nations. They are indispensable in creating the political and operational preconditions for a functional and sustainable regional power exchange.

Examples of Inter-governmental agreement/ MoU, and detailed agreements/ procedures/ guidelines/ rules that govern the power trade: - SAPP’s intergovernmental and inter-utility MoUs, and Operating Guidelines; - Marco treaty and Regional Electricity Market Regulations (REMR) in Central America; - GCC General Agreement and Power Exchange and Trading Agreement (PETA), for GCCIA; - GMS Inter-Governmental Agreement (IGA) on Regional Power Trade; - Memorandum of Understanding on the ASEAN Power Grid; and - Directives, rules and guidelines of European Commission for European common market

### Common working language

A common working language is a fundamental non-technical requirement for the successful establishment and operation of a regional power exchange, as it underpins effective communication and collaboration among member countries. Regional power exchanges typically involve diverse stakeholders such as utilities, grid operators, regulators, policymakers, and market participants from different linguistic and cultural backgrounds. The adoption of a shared working language, such as English in many international frameworks, facilitates the harmonization of technical standards, regulatory processes, and operational procedures. This alignment is critical for ensuring smooth coordination, particularly in real-time operations, where clarity and speed of communication are essential to prevent misunderstandings or delays.

Moreover, a common language serves as a platform for capacity building and knowledge transfer. Training sessions, technical workshops, and joint studies conducted in a mutually understood language enable stakeholders to share expertise, adopt best practices, and foster innovation. For instance, in the Southern African Power Pool (SAPP), English is used as the standard language for communication, allowing member states with different native languages to collaborate effectively on complex energy projects and market operations. Similarly, Nord Pool leverages a common language framework to coordinate its extensive cross-border market operations seamlessly. In addition to operational benefits, a shared language also aids in fostering trust and transparency, which are crucial for long-term cooperation. It ensures that all parties have equal access to information, enabling informed decision-making and reducing the potential for disputes. Ultimately, the implementation of a common working language as part of political agreements not only streamlines operational and regulatory activities but also strengthens the foundation for enduring regional energy cooperation.

A common working language becomes especially critical in the BBINS context, given the linguistic diversity and varying levels of English proficiency among the countries. The adoption of a shared working language likely English, which is widely used in international and regional cooperation can address potential barriers to communication and coordination. This is vital for ensuring the smooth implementation and operation of a regional power exchange, where real-time data sharing, operational alignment, and technical negotiations require unambiguous understanding. The common language will also support the development and enforcement of harmonized market rules, standardized contracts, and shared operational protocols. For instance, it will be critical for drafting technical guidelines, resolving disputes, and facilitating cross-border agreements. In addition, using a common language can bolster capacity-building initiatives, such as regional training programs and workshops, allowing stakeholders across countries to build a shared understanding of market mechanisms and operational practices. These efforts have been effective in similar setups like the ASEAN Power Grid, where English serves as the common platform for multilateral engagement.

## Technical

The technical requirements for a regional power exchange majorly encompass the essential rules and procedures needed to enable secure and efficient cross-border electricity trading. These requirements are designed to harmonize technical and operational aspects, ensuring seamless integration and coordination across national grids. Harmonized grid codes are fundamental to ensuring that power system operations across interconnected networks are compatible and free from conflicts. These codes establish uniform protocols for frequency regulation, voltage stability, contingency handling, and other critical aspects, reducing operational risks and enhancing reliability. Similarly, a standardized methodology for wheeling charges is crucial to facilitate trading between any participating member countries, irrespective of whether they share a physical border. This ensures equitable and transparent cost allocation for using transmission infrastructure, enabling greater trading flexibility.

Data and information sharing agreements are equally critical for regional power market operation. Such agreements must clearly define the data categories to be shared and the data to remain private. For instance, sensitive information such as existing contracts or details of critical infrastructure should remain confidential at the national level. However, regionally relevant data, such as dispatch schedules, generator-specific bids, and offers must be accessible to market participants to support trading. Additionally, publicly available data on transmission capacity, historical cross-border flows, and average clearing prices enhances transparency and builds trust among stakeholders. Interconnector capacity calculation methodologies are another technical cornerstone. These ensure optimal utilization of cross-border transmission lines while maintaining grid security. Clear and standardized approaches to capacity calculation prevent congestion and enable efficient electricity trading.

In practice, harmonizing these technical elements requires robust institutional mechanisms, cooperation among member countries, and adherence to agreed-upon rules. Together, these technical requirements create the foundation for a sustainable and well-functioning regional power exchange, enabling the BBINS region to realize its potential for integrated energy markets. Major technical requirements with respect to setting up/ operating of a regional power exchange are provided as follows:

### Harmonized grid codes

Harmonized grid codes are foundational for the development and operation of a regional power exchange, ensuring that interconnected power systems operate in a secure, reliable, and efficient manner. These codes establish common rules and procedures across member countries, addressing differences in operational standards and design criteria that could otherwise compromise the stability of the regional grid. Harmonization focuses on aligning key technical and operational elements to enable seamless cross-border electricity trading without imposing the need for complete unification of national grid codes. At its core, harmonization aims to address security risks inherent in interconnected systems. A unified set of regional standards can mitigate vulnerabilities from external shocks, such as sudden outages or unexpected surges in demand, which could ripple across borders. While national grid codes govern the internal dynamics of a country’s power system, harmonized regional grid codes apply to interconnectors—critical transmission links that facilitate cross-border power flows. These codes ensure that interconnected grids maintain compatibility in parameters such as voltage levels, nominal frequency, protection schemes, and contingency protocols, particularly in synchronous systems like those between India-Bhutan and India-Nepal. In contrast, asynchronous interconnections, such as the HVDC back-to-back systems between India and Bangladesh, inherently reduce risks of fault propagation between grids. However, even in such systems, the tripping of an HVDC terminal can create significant disturbances, including loss of load or supply. Future interconnections, such as potential AC links between India and Bangladesh or HVDC systems with Sri Lanka will require careful planning to ensure operational compatibility, particularly given the technical and economic considerations of these projects.

Real-time operational coordination is indispensable for maintaining grid stability and enabling efficient cross-border trade. This involves establishing dedicated communication channels, such as operator hotlines, and creating robust frameworks for data and information sharing. Sharing operational data, such as dispatch schedules, interconnector capacity, and cross-border flow records, is critical for coordinated operations and market transparency. However, data sensitivity must also be managed carefully. While confidential information, such as infrastructure details and bilateral contracts, can remain protected, access to regionally relevant data must be ensured for stakeholders to participate effectively in trading. Publicly available data, such as historical power flows and average clearing prices, further enhance transparency and foster trust. Joint emergency response and recovery protocols are another crucial component of harmonized grid codes. Grid operators from interconnected systems must collaborate on contingency planning to address potential disruptions. These protocols ensure rapid and coordinated responses to system failures, minimizing the impact of such events on regional stability.

Beyond the technical and operational aspects, harmonized grid codes play a strategic role in fostering trust among member countries. They provide a common framework that ensures equitable and reliable operations, strengthening collaboration and reducing uncertainties in cross-border trading. Thus, the harmonization of grid codes is not merely a technical requirement but a strategic enabler for regional power market integration wherein by addressing compatibility challenges, enhancing coordination, and building trust, harmonized codes form the backbone of secure and efficient regional electricity trading. For the BBINS region, such harmonization will be instrumental in unlocking the full potential of cross-border energy cooperation, driving economic growth, and ensuring energy security for all participating nations.

Case Study: RPTCC The Regional Power Trade Coordination Committee (RPTCC), which manages regional power trade in the Greater Mekong Subregion undertook the development of a harmonized grid code with the support of the Asian Development Bank (ADB). This initiative aimed to align the national grid codes of the Greater Mekong Subregion (GMS) member countries. The harmonization process has successfully addressed various technical gaps, ensuring greater consistency and compatibility across the national grid codes. As a result, the member countries have now adopted the GMS grid code, fostering improved technical integration and facilitating smoother cross-border power trade within the region

### Harmonized wheeling charges

As has been already established, harmonizing technical standards is a critical aspect of establishing and operating a regional power exchange. Equally important is the agreement on the commercial terms for transmission services provided by wheeling (or transit) countries. Wheeling charges, which compensate the transmission owner for the service, must be clearly defined and fairly structured. These charges should be paid by the transmission system user to the transmission owner responsible for providing the wheeling service. A relevant example can be found in the Lao PDR-Thailand-Malaysia-Singapore Power Integration Project (LTMS-PIP), where Lao PDR, as the generator, pays the wheeling charge to Malaysia, the wheeling country. Lao PDR then recovers the charge from Malaysia under the terms of their bilateral Power Purchase Agreement (PPA). The wheeling charges should reflect the full costs incurred by the transmission owner in providing the necessary transmission infrastructure and services. These costs encompass fixed and variable expenses, including capital investment, asset depreciation, operational and maintenance costs, as well as any losses associated with the service. The structure of wheeling charges may vary across different regions, but it is crucial that the methodology considers the local circumstances of each member country. Specifically, the transmission charge framework should promote the optimal use of available infrastructure, ensuring efficient flow of electricity across borders. In some instances, the internal transmission grid of a country may not be sufficiently developed to handle large quantities of wheeled power. In such cases, it is in the best interest of all parties to ensure that the country receives adequate compensation to incentivize the development of its infrastructure. This will help ensure that the transmission system can support the growing demand for cross-border electricity trade while encouraging investment in the necessary upgrades. Additionally, transmission owners should be provided with a reasonable return on investment, particularly as the utilization of existing assets increases due to cross-border electricity flows.

The methodology for wheeling charges must also account for the varying technical capabilities and available resources of the transmission system operators (TSOs) in different member countries. This includes the differing levels of infrastructure development, technical expertise, and human resources. A one-size-fits-all approach may not be feasible, as each country may have unique challenges and requirements based on its specific grid conditions and capabilities. Therefore, when setting up a regional power exchange, it is essential to create a harmonized and flexible wheeling charge framework that encourages efficient power transmission while promoting infrastructure development and ensuring fair compensation for transmission service providers. A balanced approach will contribute to the long-term sustainability of the regional power market, ensuring that all parties are adequately incentivized to maintain and upgrade their transmission infrastructure, ultimately fostering a reliable and integrated electricity market.

A wheeling arrangement needs to consider both the physical and commercial aspects of transmission of power. Some of the key aspects that need to be considered in a basic arrangement can be summarized as follows:

Figure 13: Basic considerations for harmonization of wheeling charges

It is possible for this methodology to cover both long-term bilateral trading agreements and short-term trading.

Design parameters that may be taken into account for developing appropriate wheeling methodology are as follows:

- Complexity level: As a starting point, the methodology should be simple to use and agreed upon (e.g. one with a minimum level of required harmonization and implementation effort). It is possible to develop more detailed/advanced methods at a future stage.

- Reflective of the cost of grid use: The wheeling charges could follow general rules that approximately reflect actual grid costs, or they could reflect the actual effect each trade has on the relevant parts of the transmission system’s infrastructure. The latter methodology is more accurate from a system perspective, but also more complex to implement.

- Cost recovery: As a general rule, wheeling charges should seek to cover the fixed and variable costs of providing the transmission services, including a reasonable rate of return for the transmission owners.

- Wheeling price certainty: Price stability is important from the perspective of investors and other stakeholders seeking to develop generation or transmission projects. However, this should not preclude changes to the wheeling methodology over time.

- Transparency: A transparent methodology should be favoured, as this builds trust among relevant stakeholders. Transparent in this context refers both to the development of the methodology itself (including any future changes), which should be done in an open, inclusive manner, and to the determination of the wheeling charges themselves.

- Loss handling: Each trade wheeled through a transmission system will see losses. Only losses directly related to the wheeled power should generally be covered by the methodology. It may be worth developing a methodology that incentivizes TSOs to minimize their losses, but this is not necessary as a first step.

- Non-discriminatory practices: All involved parties should have the same rights and obligations in the regional market. This is partly covered by the third-party agreements package but it is worth reviewing for the wheeling charges, too.

- Decentralized versus centralized management of charge collection and distribution: As a starting point, market participants should pay the wheeling charge directly to the relevant transmission owner (that is, in a decentralized manner). In the future, collection and distribution of these charges could be managed by a centralized institution.

- Possibility for a methodology applicable to both international and domestic transmission services: It is possible for a wheeling charge methodology to also be used to determine domestic transmission charges. This is not a requirement, but some countries may find it beneficial, and so it should at least be discussed at an early stage.

The stability of wheeling charges is a critical factor in facilitating predictable costs for potential investors in new generation and transmission projects. A stable wheeling charge methodology ensures confidence among stakeholders and supports the long-term viability of regional power trading frameworks. However, the wheeling charge methodologies adopted by member countries should retain the flexibility to evolve over time. For instance, advanced methodologies that calculate charges based on the actual impact of a trade on the transmission system using load flow analysis to determine specific costs, can be introduced at a later stage. While this approach provides greater accuracy in reflecting the cost of transmission services, it is complex and may not be suitable as an initial model for regions like BBINS. In the near term, minimal harmonization of wheeling methodologies across BBINS may suffice to enable trading. Although regional harmonization simplifies multilateral trading, it is not an immediate prerequisite. The primary focus should be on ensuring that the methodology is transparent in both its development and application. A practical alternative is to allow for country-specific wheeling methodologies, with clear communication to all stakeholders regarding the applicable methodology in each jurisdiction. Under this approach, wheeling charges can be paid directly to the respective national transmission system operators (TSOs). If a regional institution is established in the future, it could take on the responsibility of calculating, collecting, and distributing wheeling charges thus making case for formation of necessary institutional framework for facilitating cross-border power trading.

Wheeling charge methodologies also play a pivotal role in incentivizing transmission owners to provide capacity for multilateral trading by establishing predictable compensation mechanisms. Over time, these methodologies can be refined to become more sophisticated or harmonized, in line with evolving market models. Changes in market dynamics may necessitate corresponding adjustments to how transmission owners are compensated, ensuring alignment with regional power exchange objectives and the equitable sharing of costs and benefits.

Case Study Energy Purchase and Wheeling Agreement (EPWA) was signed by Electricite du Laos (EDL), Electricity Generating Authority of Thailand (EGAT) and TNB (Tenaga Nasional Bhd) of Malaysia at the 35th AMEM held in Manila in September 2017, for implementation of Phase 165. The EPWA would govern the obligations of the parties in the selling, wheeling and purchase of energy in the project. The duration of these contracts was two years and can be renewed. The agreement is flexible, in the sense that in case of excess power, it can be sent back to Thailand. It includes three components – commercial, technical and wheeling charge. The EPWA covers the following key aspects of the LTMS-PIP: - Energy payments from TNB (Malaysia) to EDL (Laos) - Wheeling charge payments from EDL (Laos) to EGAT (Thailand). These are payments made by the energy seller to the wheeling service provider only. - Transit Charges

### Third-party access

Facilitating third-party access (TPA) to domestic transmission grids is a critical technical requirement for the establishment and operation of a regional power exchange in BBINS. In regions where transmission infrastructure is primarily owned and operated at the national level, enabling multilateral power trade presents challenges. These include determining which entities should be allowed to trade across borders and how external stakeholders can access and utilize domestic grids. Transparent and non-discriminatory principles are essential to ensure that all market participants, including vertically integrated utilities and independent power producers (IPPs), have equitable access to the transmission network.

The potential role of privately owned cross-border transmission lines must also be considered. Some of these interconnectors, originally built for dedicated export arrangements, could be integrated into national or regional grids to support multilateral trading. Transferring the ownership of such lines to a transmission company or regional entity could help standardize access and operational principles. This would prevent market power abuse and ensure compliance with agreed guidelines for future interconnector capacity sharing and interconnection terms.

Regional TPA differs from domestic TPA. While the latter involves granting third-party domestic resources access to the local grid, the former focuses on allowing external market participants from other jurisdictions to utilize domestic transmission infrastructure. Although domestic TPA is not mandatory for regional multilateral trade, establishing a regional TPA framework is necessary to ensure the seamless flow of power across borders. Agreements among participating countries must address how interconnection and capacity rights are shared, managed, and operated transparently, laying the foundation for a robust and inclusive regional power exchange. A high-level overview of the basic agreements required is provided as follows:

- Transmission license: This agreement sets out the TPA provision for the transmission infrastructure owner and, if a different entity, the TSO of the grid. It states that these stakeholders needs to provide TPA based on certain principles to allowed parties when they request access to and use of the grid.

- Connection and usage agreement: This agreement covers the requirements for generators and, if appropriate, large consumers to be allowed to connect to and use the transmission grid. This requires, among other things, compliance with both national and regional grid codes, and that generators obey with the operational minimum requirements.

- Potential additional licenses required for generators to operate: Minimum contractual requirements to operate as an independently owned and operated generator (IPP). This license may differ among countries, and some countries may choose not to allow IPPs to participate directly in cross-border power trade. In essence, however, a generation license and an export/import license would be necessary.

The agreements should comprehensively define the rights and associated obligations of each participant involved in cross-border trading and operations. These obligations may include performance requirements, such as adherence to grid codes for generation units (including IPPs), as well as specific operational standards tailored to certain technologies or areas. For transmission operators, the primary responsibility lies in maintaining the secure and reliable operation of the grid, ensuring voltage levels and frequencies remain within safe operational limits. Additionally, they are required to provide access to their grid for any external participant authorized to engage in cross-border electricity trading. The involved parties and their respective rights and obligations can be summarized as follows:

Figure 14: Rights and Obligations of involved parties in Third Party Access (TPA)

### Data and information sharing

Effective multilateral power trading requires robust mechanisms for data and information sharing among stakeholders. A central coordinating institution can play a pivotal role by receiving, processing, and distributing necessary information through a centralized IT and communications platform. For instance, such an entity could gather transmission system data from transmission owners and relay critical information, such as traded volumes, back to relevant stakeholders. However, sharing operational and planning data across borders may raise concerns over confidentiality, as some data might be deemed too sensitive for open exchange. In certain cases, existing legislative or regulatory frameworks may restrict data sharing, necessitating changes to national laws or the inclusion of specific exceptions to allow information exchange with a regional coordinating body.

Initially, data sharing requirements for the regional power exchange are likely to be minimal, focused on enabling bilateral or multilateral exchanges among a limited group of participating countries. These countries can determine specific data needs and establish secure mechanisms for sharing. As the market matures and becomes more organized, data sharing practices and requirements should be harmonized to support more advanced trading arrangements.

At a minimum, data-sharing agreements should cover essential aspects of cross-border power trading, such as aggregate or calculated Available Transfer Capability (ATC), cross-border power flows, and system-planning information. Future regional market operations may require a central institution to manage databases, optimize power system operations, acquire and process metering data, calculate trade settlements, and distribute information via online platforms. Importantly, agreements defining data-sharing requirements should be flexible and easy to amend, allowing them to evolve alongside the regional power market.

### Interconnector capacity calculation methodology

The efficient and equitable allocation of interconnector capacity is a critical technical requirement for enabling cross-border power trade and supporting the operation of a regional power exchange. To ensure consistency and transparency, interconnector capacity should ideally be calculated using a harmonized methodology agreed upon by all parties on both sides of an interconnected border. Such harmonization fosters mutual trust, reduces disputes, and allows for effective coordination across jurisdictions. In cases where agreement on a common methodology is not immediately feasible, a conservative approach can be adopted by using the smaller of the independently calculated capacities from each side. This ensures system reliability and provides a basis for gradually aligning methodologies in the future.

Interconnector capacity should also be allocated across multiple time frames, including long-term, medium-term, and real-time horizons. Long-term allocations provide stability and predictability for market participants, enabling better planning and investment decisions. Medium-term and real-time allocations, on the other hand, facilitate flexibility and the optimal utilization of available capacity as grid conditions and market dynamics evolve. These allocations are particularly important for jurisdictions aiming to trade capacity products, such as transmission rights, alongside energy products.

By adopting a well-structured capacity calculation and allocation framework, member countries can optimize interconnector usage, enhance system reliability, and support the development of an efficient and integrated regional power market.

Case Study: Pentalateral Energy Forum - Resource Adequacy Assessment The Pentalateral Energy Forum (PLEF) facilitates regional cooperation in electricity market integration and security of supply across Austria, Belgium, France, Germany, Luxembourg, the Netherlands, and Switzerland. A key initiative, the Pentalateral Generation Adequacy Assessment (PLEFGAA), highlights the importance of harmonized interconnector capacity calculation methodologies for regional power markets. The third edition of the PLEFGAA employed a probabilistic approach to jointly assess generation adequacy and security of supply across the PLEF region. By standardizing capacity calculation practices among the Transmission System Operators (TSOs) of member countries, the assessment ensured efficient and transparent utilization of interconnectors. It also allocated capacity across multiple time frames—real-time, short-term, and long-term—balancing reliability with flexibility to optimize infrastructure usage. The PLEF experience demonstrates how harmonized methodologies reduce uncertainty, enhance cross-border collaboration, and ensure predictable access for market participants. This approach serves as a practical model for regions like BBINS, highlighting the value of regional cooperation in addressing technical challenges and strengthening interconnected power systems.

## Institutional Arrangements

The political and technical requirements discussed so far such as harmonized regulations, standardized interconnector capacity calculations, third-party access mechanisms, and transparent data-sharing protocols, serve as critical levers for identifying and addressing institutional needs. These requirements underscore the necessity of institutions that can oversee compliance, manage complexities, and foster trust among participating nations. Institutions must have the capacity to translate these requirements into actionable frameworks, ensuring the seamless operation of a regional power exchange.

Establishing a regional power exchange requires robust institutional arrangements/ mechanisms to support the transition from bilateral to multilateral trading. This includes both strengthening existing institutions and potentially creating new entities with specific responsibilities to ensure effective market organization and coordination. For regions like BBINS, this transition will necessitate additional capacity building at existing institutions, such as Transmission System Operators (TSOs), and the establishment of new institutions to manage functions such as settlement, payment mechanisms, and dispute resolution.

International experiences highlight that regional institutions play a crucial role in supporting multilateral trading and regional integration. For instance, institutions like PJM and ISO New England in the United States, or the European Network of Transmission System Operators for Electricity (ENTSO-E) in Europe, help oversee regional integration and enforce market rules. Similarly, India’s Central Electricity Regulatory Commission (CERC) and the Agency for the Cooperation of Energy Regulators (ACER) in Europe provide regulatory frameworks that guide power markets across national boundaries.

In the case of secondary power trading arrangements, regional institutions further contribute to efficient energy coordination. For example, the Southern African Power Pool (SAPP) operates under the Southern African Development Community (SADC), while the SIEPAC system involves multiple institutions including a regional regulator and an operating entity. These cases show that dividing responsibilities between institutions can lead to more efficient governance and equitable distribution of tasks across participating countries.

For BBINS, the evolution towards a regional power exchange will require leveraging existing institutions while simultaneously establishing new ones tailored to multilateral trading. These institutions will not only oversee trading but will also manage critical infrastructure, ensuring seamless coordination, regulatory compliance, and market efficiency. The successful transition will combine both top-down institutional support and bottom-up country-driven needs, building on the foundation of existing bilateral arrangements.

### Models

Institutional models for regional power exchanges vary based on the desired level of coordination and integration. They range from platforms that foster collaboration among national regulators to fully unified institutions managing all aspects of cross-border trade. The choice of model depends on regional priorities, existing institutional frameworks, and the complexity of market operations. These models provide a framework for organizing regulatory, operational, and market functions to support efficient and transparent regional power trading.

- Regional Platforms for Energy Regulators

Regional platforms for energy regulators serve as mechanisms for coordination and cooperation among national regulatory authorities involved in cross-border power trading. These platforms operate in the absence of, or alongside, supra-jurisdictional regulatory structures, providing a forum for harmonizing practices, exchanging information, and addressing regulatory challenges. They facilitate alignment on key issues such as grid codes, tariffs, and market operations while preserving the sovereignty of national regulators. Initiatives like these also support capacity building and the sharing of best practices among participating countries. For example, platforms such as the Council of European Energy Regulators (CEER) and the ASEAN Energy Regulatory Network (AERN) enable collaboration and knowledge-sharing on regulatory matters in their respective regions. Similarly, South Asian forums like the South Asia Forum for Infrastructure Regulation (SAFIR) and the Council of Experts of Energy Regulators (CEERE) focus on cross-border trade operations and regulatory alignment.

Despite these efforts, the success of such platforms in fully harmonizing rules and regulations for multilateral power trading has often been limited, underscoring the challenges of achieving deep integration under this model. Nevertheless, regional platforms remain a valuable starting point for fostering trust and cooperation among regulators and setting the stage for greater regional integration in the future.

- Central Coordinating Entity

Regional interconnected power systems can operate effectively without assigning traditional regulatory authority to a single regional body. Instead, a central coordinating entity typically takes on a facilitative role, working among local regulators and stakeholders to harmonize practices, manage operations, and support market integration. This model avoids centralizing binding regulatory authority while fostering collaboration and coordination among participating countries.

The Southern African Power Pool (SAPP) exemplifies such a framework, where the regional association RERA (Regional Electricity Regulators Association) plays a limited role in advancing regulatory harmonization and capacity building. As SAPP operates as a self-regulated market, participating countries must collectively agree on market and transmission rules. RERA’s mandate does not include enforcing regulations but focuses on supporting the development of SAPP. This demonstrates that market development can progress without granting significant authority to a central entity, although RERA’s limited mandate has constrained harmonization in critical areas such as transmission planning and operating reserves.

In regions adopting this model, the central coordinating entity undertakes key responsibilities such as managing interconnector capacity allocation, coordinating grid operations, and facilitating trade settlements. While regulatory authority remains with national institutions, the entity ensures alignment across borders by serving as a neutral platform for negotiation, data sharing, and technical coordination. This approach balances national sovereignty with regional collaboration, providing a foundation for advancing multilateral power trade while addressing operational challenges.

Efforts to expand the mandate of such entities, as seen in ongoing discussions about RERA, highlight the potential to further streamline and harmonize regulatory and operational frameworks in regional markets.

- Centralized Regional Entity

A centralized regional entity represents an advanced institutional model for governing regional power markets. In this configuration, a regional regulatory authority coexists with local regulatory authorities (LRAs), with the overarching regional institution granted significant authority to regulate and enforce market rules. The regional entity typically manages key aspects of market operation, such as transmission planning, interconnector capacity allocation, dispute resolution, and trade settlement, ensuring uniformity and efficiency across jurisdictions.

The balance of authority between regional and local regulators varies across regions. For example, in the European Union, ACER (Agency for the Cooperation of Energy Regulators) coordinates the work of LRAs on cross-border interconnection issues, supports harmonization, and steps in when disagreements arise. However, ACER lacks binding regulatory authority, as LRAs retain control over their national systems and markets. By contrast, in the United States, the Federal Energy Regulatory Commission (FERC) functions as a federal regulator with binding authority over interstate transmission and wholesale electricity transactions, while LRAs oversee state-level distribution systems and retail markets.

Figure 15: Division of regulatory authority in the United States and the European Union

In Central America, the Regional Commission on Electrical Interconnection (CRIE) regulates the regional Mercado Eléctrico Regional (MER). CRIE oversees transmission tariffs, approves regional network extensions, ensures competition, and intervenes to prevent market abuses. It also monitors system exchanges and ensures compliance with regional performance standards. Similarly, in West Africa, the Economic Community of West African States (ECOWAS) Regional Electricity Regulatory Authority (ERERA) regulates cross-border trade, transmission tariffs, and bilateral contracts, issuing binding rules for market operation. However, the development of unified markets, such as in West Africa, has been slow due to political challenges and limited consensus among member states.

The centralized regional entity model enables significant regional market integration by consolidating oversight and facilitating harmonization. However, its success depends on strong political will, clear mandates, and effective collaboration between regional and local regulators.

- Unified Institution

The unified institution model represents the most integrated approach to regional power market regulation, where a single entity assumes comprehensive regulatory and operational oversight. This model minimizes complexity, reduces compliance costs, and ensures consistent governance across interconnected jurisdictions. The Australian Energy Regulator (AER) exemplifies this model. Established in 2005, AER governs the National Energy Market spanning five Australian jurisdictions, managing wholesale and retail electricity markets as well as electricity and gas networks. By consolidating the functions of previous regulators, AER has streamlined operations and regulatory compliance across regions. However, such a model may not always be feasible. In Canada, for instance, regulatory authority remains largely decentralized, with provinces retaining jurisdiction over energy and natural resources. While the creation of the Canada Energy Regulator (CER) in 2019 modernized oversight for interprovincial and international power lines, it did not establish a unified national framework, reflecting the provinces’ emphasis on sovereignty and independence.

In cross-border contexts, the creation of a unified institution faces additional challenges, including sovereignty concerns and the necessity for national regulators to retain oversight within their territories. As a result, no fully unified cross-border institution has yet been implemented, though the model serves as an aspirational benchmark for regions aiming to achieve high levels of integration.

Institutional Development in South Asia In South Asian context, South Asia Regional Energy Partnership (SAREP) is working to create “South Asia Network for Integrated Power System Planning and Operation (SANIPO)“, a regional level platform to facilitate capacity building, knowledge exchange and sharing of best practices related to Integrated Power System Planning and Operation.

### Settlement and payment mechanism

An efficient and secure settlement and payment mechanism is a cornerstone of successful multilateral power trade in a regional power exchange. This mechanism ensures that the financial aspects of trading; such as payments for traded volumes, wheeling charges, loss settlements, and other monetary flows are handled transparently and reliably. A central clearinghouse or central counterparty (CCP) is commonly employed to intermediate transactions, reducing overall counterparty risk in the market.

The CCP typically manages key financial operations, including the collection and posting of collateral from market participants to mitigate market risks. Collateral requirements are calculated based on participants' trading activities and potential exposure and are critical to maintaining market integrity. Additionally, the clearinghouse oversees the timely settlement of transactions, which is essential for maintaining confidence in the trading system.

Potential role of Central Counterparty:

- responsibility for all clearing-related operations for electricity wholesale markets

- interface to the market participants

- interface to the market operator settlement team (if separate entity)

- internal data provider for financial reporting services

- partial owner of the operational procedures for wholesale electricity markets clearing (with

- ownership shared with the relevant transmission owners, which provide relevant data and retain ownership of that data).

The purpose of a CCP is to facilitate a transparent and efficient flow of funds while managing the financial settlement and clearing of all trades within the organized market(s). Additionally, a CCP mitigates counterparty risk by stepping in to fulfill obligations and cover any monetary shortfall in the event of a market participant's failure to meet its commitments

Figure 16: Overview of CCP Function

Source: Nord Pool

To support this mechanism, member countries participating in the regional exchange must agree on minimum requirements for settlement and clearing functions at the outset. These agreements should cover operational timelines, collateral policies, and the roles and standards for any banks involved in processing financial transactions. Given the larger volume of transactions in organized markets, rapid settlement timelines are particularly important to ensure market liquidity and reduce financial risk.

By establishing a robust settlement and payment framework, regional power exchanges can ensure smooth financial operations, build trust among participants, and foster the long-term sustainability of cross-border power trade.

### Dispute resolution mechanism

Establishing a robust dispute resolution mechanism is critical for the effective functioning and development of multilateral power trading in a regional power exchange. Disputes may arise during actual trading, such as failures in delivery, unintended cross-border exchanges, or payment-related disagreements. Alternatively, disputes may emerge during the process of setting up multilateral trade frameworks, which are crucial to ensuring that development progresses without losing momentum. Depending on the nature and origin of disputes, it may be necessary to implement multiple mechanisms tailored to address specific types of conflicts.

The establishment of these mechanisms, and any associated institutions, should be prioritized to ensure smooth market operations and avoid gridlocks in the development process. It is essential that all stakeholders recognize the authority of the dispute resolution framework, which may require formal agreements at the intergovernmental level. A unified and widely accepted mechanism will help ensure consistency in resolving disputes across the region and maintain cohesion in the methodologies and rules governing multilateral power trading. Such mechanisms serve as a foundation for fostering trust and cooperation among participating entities, ultimately supporting the long-term success of regional power exchanges.

International best practices - SAPP, WAPP, GMS, Central America: dispute resolution procedures are part of inter-country agreements & treaties. Also part of bilaterally negotiated PPAs. - Most preferred method of dispute settlement resolution is amicable settlement between the parties. - The last resort of settlement is through international arbitration (UNCITRAL, ICSID, etc.) as mentioned in the agreements/treaties

Having outlined the minimum political, technical, and institutional requirements necessary for establishing and operating a regional power exchange, the focus now shifts towards the market platforms and the various products that support multilateral trading.

# 1 Market platforms and products, including derivatives

The development of market platforms and their associated products is essential for the efficient operation of modern power markets. These platforms enable price discovery, liquidity, and risk management, which are critical for balancing supply and demand in dynamic electricity markets. They ensure transparency and provide mechanisms for market participants to engage in efficient transactions. Market platforms typically offer physical contracts for the purchase and sale of electricity, along with financial instruments such as futures, options, and swaps. These derivatives are crucial for managing price volatility, a common challenge in electricity markets due to fluctuating weather, fuel prices, and regulatory changes. By using these products, market participants can hedge against risks and optimize their strategies, ensuring price stability and market efficiency. In the context of a regional power exchange, these products facilitate cross-border electricity trade and integration of complementary energy resources. For instance, the BBINS region can leverage hydropower from Bhutan and Nepal alongside thermal and renewable energy from India and Bangladesh, improving grid stability and addressing demand variations. This integration also offers economic benefits, including reduced reliance on imports and more competitive electricity prices. Derivatives, in particular, help mitigate the risks of cross-border trading, such as currency fluctuations and price volatility, ensuring a stable and liquid market. The presence of these financial instruments enhances market depth and attracts investment, while supporting price formation and risk management. A well-functioning regional power exchange, underpinned by diverse market products, can optimize resource use, reduce costs, and promote cleaner energy. Such exchanges contribute to energy security, sustainability, and a more resilient regional power system. This chapter explores the role of different market platforms and products, including derivatives, in fostering efficient, transparent, and sustainable electricity markets, where regional power exchange can serve as a key application of these mechanisms.

## Regional Power Exchange

A regional power exchange is a platform that facilitates the trading of electricity between neighboring countries or regions, enabling the efficient allocation of energy resources across borders. By integrating the electricity markets of multiple countries, such an exchange allows for the optimal use of available power, helping balance supply and demand more effectively. It supports price discovery, increases market liquidity, and provides participants with access to a wider range of energy sources, leading to cost-effective electricity procurement. Additionally, regional power exchanges often offer financial products, such as futures and options, to help manage price volatility and mitigate market risks. The exchange of electricity across borders stabilizes regional grids, enhances energy security, and promotes the use of cleaner, more sustainable energy sources. For regions like BBINS, where countries have complementary energy resources, a regional power exchange could enhance cooperation, optimize resource sharing, and improve grid stability.

### International Case Studies

Nord Pool

The development of the Nordic electricity market was driven by the complementary energy resources of the Nordic countries. Norway and Sweden primarily generate electricity from hydropower, while Finland and Denmark rely largely on thermal power systems. This synergy has led to greater economic efficiency, enhanced security of supply, and improved environmental performance in the Nordic power system. The integration of Swedish and Norwegian hydropower has resulted in lower average electricity prices and reduced carbon emissions across the region. Additionally, the inclusion of thermal capacity from Denmark and Finland has further strengthened security of supply, especially during dry years.

The Nordic countries have a long-standing tradition of cooperation and trade between their national electricity systems, formalized in 1963 with the establishment of Nordel (Nordic Electric Power System Coordination group). This collaboration led to increased technical cooperation and trade, culminating in the creation of a common Nordic electricity market. In 1995, the Nordic Ministers of Energy issued the Louisiana Declaration, recognizing that a pan-Nordic electricity market would bring economic and environmental benefits to all countries and should be established promptly. The market integration unfolded in phases, beginning with the creation of the Nord Pool exchange in 1996, which initially covered Norway and Sweden. Finland joined in 1998, and Denmark followed in 2000, completing the day-ahead spot market for the entire Nordic region. Today, all consumers can choose their electricity retailer, and all retailers and generators have access to the market. The integration process also involved the removal of barriers to cross-border trade, the introduction of common grid codes, joint management of reserves, coordinated planning of interconnectors, congestion management, harmonized balance settlement, and unified goals for retail markets, all of which have contributed to a more efficient and secure electricity system across the Nordic region. Major developments of Nord Pool over the years have been presented as follows:

Table 2: Evolution of Nord Pool

| Timeline | Development |
| --- | --- |
| 1991–1995 | Norwegian power market deregulated |
| 1993 | Nord Pool Spot established as ‘Statnett Marked’ |
| 1995 | Nordic energy ministers formed a committee to investigate the establishment of a common Nordic electricity market. Committee provides favourable report. |
| 1996 | Rebranded to Nord Pool when Sweden joins |
| 1998 | Finland joins Nord Pool |
| 1999 | Elbas becomes the first intraday market |
| 2000 | The Nordic market fully integrated as Denmark joins |
| 2001 | Independent Market Surveillance is established |
| 2002 | Nord Pool Spot established as a separate company |
| 2005 | The Kontek bidding area in Germany opens |
| 2008 | Financial market sold to NASDAQ OMX Commodities |
| 2009 | Market coupling of 11 European countries launched through European Market Coupling Company (EMCC) |
| 2010 | N2EX launched by Nord Pool Spot and NASDAQ OMX Nord Pool Spot opens bidding area in Estonia |
| 2012 | Nord Pool Spot opens bidding area in Lithuania |
| 2013 | Nord Pool Spot opens bidding area in Latvia thus including all the Baltic countries |
| 2014 | Nord Pool Spot confirms successful takeover of N2EX Clearing and Operations |
| 2015 | New bid products for Nordic-Baltic Day-ahead auction |
| 2016 | CROPEX and Nord Pool announce development of Croatian intraday market |
| 2017 | Nord Pool and Agder Energi join forces to create the marketplace of the future |
| 2018 | Nord Pool restructuring separating its Market Coupling Operator functions from the commercial Power Exchange functions. |
| 2020 | Update to system price calculation - launch of NordLink Nord Pool announces GB Auction changes for BREXIT |
| 2021 | Inclusion of the Bulgarian-Romanian border into the Single Day-Ahead Coupling (SDAC) |
| 2022 | Launch of Flow-Based Market Coupling in the Core region |
| 2023 | Nord Pool and BRM (Bursa Română de Mărfuri S.A.) partner to deliver new Romanian Power Exchange |
| 2024 | Euronext and Nord Pool announce Nordic and Baltic Power Futures market |

Source: SARI/EI, Nord Pool

Nord Pool provides a range of services, including day-ahead and intraday trading, clearing and settlement, and additional market support, catering to businesses of all sizes and locations. With 370 companies from 20 countries actively trading on its platforms, Nord Pool operates in the Nordic and Baltic regions, Germany, Poland, France, the Netherlands, Belgium, Austria, Luxembourg, and the UK. As a Nominated Electricity Market Operator (NEMO) in 16 European countries, it also facilitates power markets in Bulgaria, Croatia, and Georgia. In 2023, Nord Pool recorded a total traded power volume of 1,103.83 TWh.

Nord Pool Spot (Nord Pool power exchange) operates the following leading markets for buying and selling power in Europe:

- Elspot day-ahead market in the Nordic and Baltic regions

- Elbas intraday markets in the Nordic and Baltic regions

- N2EX market in the UK

The day-ahead and intraday markets complement each other, with the intraday market serving as a balancing mechanism. The intraday market operates continuously, allowing power trading until 30 minutes before delivery, while the day-ahead market is conducted the day before the actual power transfer. The Elbas, a cross-border intraday market, has been in operation since 1999. The TSO market functions as a real-time balancing market, utilizing two key balancing tools: the balancing market and the ancillary services market. All products in these markets are physical contracts. In the day-ahead market, balance is achieved between generation and demand, which is fine-tuned in the intraday market. The balancing power market (BPR) helps maintain hourly balance, while the ancillary services market focuses on frequency control. The EU Target Model for energy trading operates on a unified algorithm for the day-ahead market (with a gate closure at 12:00 for the next day's 24-hour period) and a shared order book for the intraday market. This progress is driven by collaboration among eight European power market operators.

The core responsibilities of the Power Exchange are providing a price reference to the power market, operating a spot market, and organizing a market for financial products (products like forward, futures and option contracts). The power exchange acts as a neutral and reliable power-contract counterparty to market participants. It uses the spot market’s price mechanisms to alleviate grid congestion (capacity bottlenecks) through optimal use of available capacity. To this end, the Nord Pool Spot operates the leading markets to buy and sell power in Europe. This arrangement enables countries to balance electricity supply and demand more effectively across borders, enhancing energy security and optimizing resource utilization. Market integration has established a stable foundation for expanding cross-border transmission infrastructure, aligning with the European Union's goal for all Member States to achieve cross-border transmission capacity of at least 15% of their generation. These investments in transmission networks have been crucial for facilitating market integration, allowing greater volumes of electricity to flow seamlessly between countries.

Key Features of Market Structure

- One Country One Regulator: Each country in the Nordic region has its own energy regulator, as detailed below, with NordREG (Nordic Energy Regulators) facilitating regional cooperation and ACER (Agency for the Cooperation of Energy Regulators) overseeing pan-European collaboration:

- Danish Energy Regulatory Authority (DERA) – Denmark

- Energy Market Authority (Energiamarkkinavirasto) – Finland

- Norwegian Water Resources and Energy Directorate (NVE) – Norway

- Energy Markets Inspectorate – Sweden

NordREG, established through the longstanding tradition of energy cooperation among Nordic countries, serves as a collaborative body for Nordic energy regulators. Supported by the Nordic Council of Ministers, NordREG aims to advance the realization of a unified Nordic energy market, encompassing both wholesale and retail markets. Its mission is to promote the necessary legal and institutional frameworks for the development of robust Nordic and European electricity markets. ACER oversees the coordination of European energy regulators, monitors regional cooperation among transmission system operators (TSOs) in the electricity and gas sectors, and supervises the functioning of the single European electricity and gas markets. Established with legal capacity on 3 March 2011, ACER plays a pivotal role in ensuring regulatory alignment across Europe. Complementing ACER's efforts, the Council of European Energy Regulators (CEER), a not-for-profit association of national regulators, facilitates collaboration and the exchange of best practices. CEER's primary goal is to support the development of a unified, competitive, efficient, and sustainable internal energy market in the European Union, prioritizing the public interest.

- One System Operator per Country: The Transmission System Operators (TSOs) in the Nordic countries are Statnett SF (Norway), Svenska Kraftnät (Sweden), Fingrid (Finland), and Energinet.dk (Denmark). Previously, Nordel facilitated cooperation among Nordic TSOs to optimize electricity generation and transmission, serving as an advisory and market forum. In 2009, operational tasks were transferred to ENTSO-E (European Network of Transmission System Operators for Electricity) to support EU market integration, ensure supply security, and establish grid rules. Each TSO is responsible for national electricity transmission, maintaining balance between production and consumption, ensuring grid reliability, and operating neutrally and fairly toward market participants.

Regional Integration in Nordic Market

Benefits: Regional integration has brought significant benefits, particularly in optimizing the use of hydropower reservoirs and balancing generation. The introduction of sub-national bidding zones in the Nordic market allows differentiation between areas with abundant or limited water reserves, addressing local generation imbalances and highlighting transmission bottlenecks. Price differences between these zones encourage efficient power generation and signal where transmission investments are needed. Integration has also supported the inclusion of intermittent renewable energy sources, such as wind, by enabling flexible hydropower generation to balance supply. This, along with baseload nuclear power, creates a diversified and resilient system. Excess power can be exported, and during shortages, imports can be made, reducing the need for costly backup capacity and lowering overall system costs. The expanded market scale fosters liquidity and competition, keeping power prices down while ensuring a more efficient and resilient electricity system for all participating countries.

Success factors: The success of cross-border power trading in Scandinavia is due to strong regional cooperation, with ~30 years of market integration, aligned energy policies, and common standards. NordREG, TSO grid planning, and harmonized institutional frameworks have supported this collaboration. The Nordic power market now includes real-time balancing and reserve markets, which are becoming more standardized and integrated with the EU. Continued investment in grid modernization, energy storage, and European integration, along with enhanced digitalization, will be crucial to support the integration of renewable energy and growing electricity demand, ensuring the market's efficiency and reliability.

Key Learnings: - Investment in Interconnections and Grid Infrastructure: Building interconnections and upgrading grid infrastructure takes substantial time and investment. However, these efforts can yield significant benefits, such as enhanced resilience, a more efficient power system, and better progress toward decarbonization goals. - Political Will and Regional Cooperation: Successful cross-border power trading relies on strong political will and long-term regional cooperation. This involves aligning on regional grid planning and sometimes prioritizing collective benefits over short-term national interests. - Transparency and Trust in Marketplaces: Regional trading thrives on transparency and trust. In the Nordics, all market participants have equal access to price, supply, and demand information. This openness builds trust, encourages more participants, and boosts market liquidity, facilitating more effective cross-border trading.

European Power Exchange (EPEX)

EPEX SPOT is a major electricity market operator in Europe, providing a platform for the trading of electricity in various short-term markets. Established in 2008 through the merger of Powernext and the European Energy Exchange (EEX), EPEX SPOT has played a crucial role in the development and integration of European electricity markets. Major developments are highlighted as follows:

Table 3: Evolution of EPEX

| Timeline | Developments |
| --- | --- |
| 1996 | European Directive on energy market liberalization |
| 1998 | Implementation of the Directive in German and Austrian law |
| 2000 | Implementation of the Directive in French law |
| 2001 | Establishment of POWERNEXT SA |
| 2002 | Merger of the former Leipzig Power Exchange LPX and the EEX Frankfurt to EEX AG |
| 2008 | Creation of EPEX SPOT SE, owned 50/50 by POWERNEXT SA and EEX AG |
| 2009 | Creation of EPEX SPOT SE, owned 50/50 by POWERNEXT SA and EEX AG Transfer of EEX Power Spot into EPEX SPOT SE |
| 2012 | APG and EPEX SPOT launch Austrian Intraday market |
| 2014 | Launch of 15-Minute Intraday Call Auction in Germany |
| 2016 | Launch of the new Belgian-French & Belgian-Dutch Intraday solution |
| 2018 | Intraday trading harmonized across eight countries |
| 2022 | EPEX SPOT becomes first exchange to join RECS Energy Certificate Association |

Key Features

- Access to an Anonymous Market: EPEX SPOT provides participants with access to an anonymous market, ensuring that trading activities are independent of individual identities. This promotes fairness and confidentiality, allowing participants to trade without revealing their positions.

- Level Playing Field Between Members: EPEX SPOT ensures equal access for all market participants, promoting a competitive and fair trading environment. There are no preferential treatments, ensuring that all members, regardless of their size or position, have an equal opportunity to engage in the market.

- Financial Guarantees through the Clearing House: EPEX SPOT’s Clearing House offers financial guarantees to reduce the risk of default. This mechanism ensures that transactions are secured and that market participants are protected against counterparty risks, maintaining market stability.

- Calculation and Publication of Transparent and Fair Price References: EPEX SPOT calculates and publishes transparent, fair, and market-driven price references, providing clear price signals. This transparency enhances market trust, facilitates accurate price discovery, and supports efficient decision-making for participants.

Market oversight

EPEX SPOT operates a variety of electricity markets designed to promote efficient trading and ensure a reliable electricity supply across Europe. The primary products offered by EPEX SPOT include the Day-Ahead Market (DAM), the Intraday Market, and the Local Flexibility Market. The DAM allows market participants to trade electricity for the following day through a daily auction, determining hourly price signals that help balance supply and demand. The Intraday Market facilitates continuous trading, providing flexibility for participants to adjust their positions closer to real-time delivery. This is essential for managing variability in renewable energy generation and unforeseen changes in demand. Additionally, EPEX SPOT has introduced Local Flexibility Markets to address localized energy needs, enabling grid balancing in specific regions, particularly as the share of decentralized renewable generation increases.

The working model of EPEX SPOT is based on market coupling and integration. Through Market Coupling, EPEX SPOT links electricity markets in multiple countries, optimizing the use of cross-border transmission capacities and ensuring efficient price discovery. The platform is designed to be transparent, with price data being published in real-time and historical data available for market participants. EPEX SPOT also works closely with Transmission System Operators (TSOs) to ensure the smooth integration of electricity networks across regions. A key aspect of its operations is the Clearing House, which offers financial guarantees to secure transactions and mitigate risks of counterparty defaults, further ensuring market stability.

Governance Structure

EPEX SPOT operates under a two-tier governance structure, comprising the Management Board and the Supervisory Board. The company is jointly owned by EEX Group and HGRT. In addition to these boards, EPEX SPOT is supported by two other key exchange bodies: the Exchange Council and the Market Surveillance office. This governance framework ensures efficient decision-making, strategic oversight, and market integrity.

Figure 17: Governance Structure of EPEX Spot

Key Learnings: - Market Integration and Coupling: EPEX SPOT’s cross-border market coupling optimizes resource utilization and price discovery by connecting neighboring markets. Regional exchanges can adopt similar coupling strategies to enhance market efficiency. - Flexible Market Offerings: Offering Day-Ahead, Intraday, and flexibility markets enables better grid balancing and allows market participants to adjust positions in real time, especially in the context of increasing renewable energy integration. - Cross-Border Collaboration and Governance: EPEX SPOT's effective governance model and close collaboration with Transmission System Operators (TSOs) ensures smooth market operations and regional integration. - Transparency and Education: Providing real-time market data and educational resources helps build trust and ensures informed decision-making among market participants. - Adapting to Decentralized Systems: EPEX SPOT’s introduction of flexibility markets supports local grid balancing, which is increasingly vital due to the growth of decentralized and renewable energy sources.

South African Power Pool

The Southern African Power Pool (SAPP) was established in August 1995 during a summit of the Southern African Development Community (SADC) held in Kempton Park, South Africa. At this summit, SADC member governments (excluding Mauritius) signed an Inter-Governmental Memorandum of Understanding (MoU) to form an electricity power pool in the region. The Revised Inter-Governmental MoU was signed on 23 February 2006 to update the framework for cooperation. The SAPP aims to enhance regional energy security, promote economic development, and facilitate the efficient exchange of electricity across Southern Africa. Evolution of SAPP is provided as follows:

Table 4: Evolution of SAPP

| Timeline | Developments |
| --- | --- |
| 1994 | Inter-Governmental MoU signed in 1994 established SAPP |
| 1995 | 2 (Two) network links set-up via Botswana |
| 2001 | SAPP started short term market in 2001 |
| 2009 | Launched competitive electricity market - DAM |
| 2010 | Operating guidelines finalized; SAPP fully operational |

Membership & Structure

The SAPP has twelve member countries, represented by their respective electric power utilities, which are organized through SADC. The participating countries include Angola, Botswana, DRC, Lesotho, Malawi, Mozambique, Namibia, South Africa, Swaziland, Tanzania, Zambia, and Zimbabwe.

The governance structure consists of several working committees:

- Environmental Sub-Committee

- Markets Sub-Committee (established in April 2007)

- Operating Sub-Committee

- Planning Sub-Committee

These committees report to the Management Committee, which in turn reports to the Executive Committee. Additionally, the Coordination Centre Board, created in April 2007, oversees the activities of the SAPP Coordination Centre, ensuring smooth operational management. The membership includes 16 utilities, independent transmission companies, and IPPs from Angola, Botswana, Democratic Republic of Congo, Lesotho, Malawi, Mozambique, Namibia, South Africa, Swaziland, Tanzania, Zambia, and Zimbabwe. Each member contributes an annual fee as outlined in the Inter-Governmental MoU. SAPP coordinates the planning and operation of the electric power system among member utilities, providing a platform for regional solutions to electricity challenges.

Despite its success, SAPP faces challenges such as infrastructure development, managing pricing disparities, and expanding renewable energy integration. Continued investment in transmission infrastructure, energy storage solutions, and digital technologies may be leveraged to address the issues.

Key Learnings: - Regional Cooperation is Key: Successful cross-border electricity trading relies on strong regional cooperation, with member countries aligning energy policies, standards, and operational practices to ensure the smooth functioning of the power pool. - Market Integration: The development of structured markets, such as the Short-Term Energy Market (STEM) and Day-Ahead Market (DAM), facilitates price transparency, market liquidity, and efficient electricity allocation, benefiting both producers and consumers - Governance and Coordination: A robust governance structure, including agreements like the Inter-Governmental MoU and various sub-committees, ensures that member utilities work collaboratively to address challenges and make strategic decisions for the region's energy future.

### Key Takeaways from overall international experience

- Evolution pattern - The development of power exchanges often follows a gradual evolution from national markets to regional power exchanges (RPXs). Initially, a power exchange may operate within a single country, focusing on establishing the market framework, infrastructure, and ensuring system reliability. As the market matures, the focus shifts toward integrating neighboring countries, facilitating cross-border electricity trading, and creating a more competitive, liquid market. For example, the Nord Pool in Scandinavia started as a national market in Norway before expanding to include Sweden, Denmark, Finland, and eventually several other countries in Europe. The evolution involves setting up the right regulatory frameworks, creating interconnections, and ensuring that transmission capacity can handle increased cross-border flows. By gradually integrating neighboring markets, regional exchanges help optimize the use of power generation assets, reduce costs, and enhance system stability.

- Governance Structures - A regional power exchange (RPX) should have a strong, multi-tiered governance structure comprising a supervisory board, a management team, and advisory committees. The supervisory board ensures overall strategic direction and compliance with public policy, while the management team handles day-to-day operations, including market functionality and infrastructure management. Advisory committees provide specialized guidance on market design, risk management, and technological advancements. This governance framework fosters transparency, accountability, and adaptability, enabling the RPX to operate efficiently, respond to market changes, and meet the needs of diverse stakeholders in a dynamic energy landscape.

- Typical Products - The Day Ahead Market (DAM) and Intra Day Market (IDM), a variation of the Term Ahead Market (TAM), are the primary products available on national and Regional Power Exchanges worldwide.

- Settlement - A common practice for commercial settlement is through advance margins and collaterals, in accordance with the governing rules of the relevant Power Exchange

- Market Surveillance - To oversee trading activity in order to identify potential violations of market manipulation and insider trading prohibitions.

## Market Products

Setting up of regional power market will rely on a diverse set of market products to ensure efficient electricity trading, grid stability, and risk management. These products are crucial for enabling price discovery, balancing supply and demand, and maintaining system reliability across the interconnected region. The integration of the Spot Market, Day-Ahead Market (DAM), Ancillary Services/Products, and Derivatives will provide the necessary mechanisms for addressing real-time imbalances, optimizing generation schedules, ensuring grid stability, and managing financial risks. By incorporating these products into the regional market framework, regions like BBINS can foster a competitive, sustainable, and resilient energy system that benefits all participants.

### Spot

Spot markets are essential platforms in the electricity sector where power is traded for immediate or near-immediate delivery. They reflect real-time supply and demand conditions, making them highly dynamic and crucial for maintaining grid stability and integrating renewable energy sources. Unlike long-term contracts, spot markets enable market participants to respond to shifting grid conditions, such as sudden outages or variations in renewable generation, ensuring a balanced and reliable electricity supply. Spot markets may deal in the real-time, hour ahead and DAM markets.

In regional power markets, spot markets are fundamental for fostering cross-border energy trade and optimizing resource use across multiple countries. Key aspects of spot markets in regional power trading include:

- Real-Time Price Signals: Spot markets provide transparent, real-time price signals that reflect immediate grid conditions, enabling efficient allocation of electricity resources.

- Day-Ahead and Intraday Flexibility: The day-ahead market offers a structured schedule for power delivery across regions, while intraday trading allows adjustments closer to delivery, accommodating unforeseen supply-demand changes.

- Enhanced Market Liquidity: Spot markets bring together a wide range of participants, including utilities, independent power producers, and transmission operators, increasing liquidity and fostering competition.

- Renewable Energy Integration: These markets are instrumental in managing the variability of renewable sources by facilitating the exchange of surplus power and meeting short-term demand gaps.

- Grid Stability Across Borders: Spot markets enable efficient balancing across interconnected grids in regional power pools, reducing the need for costly reserves and improving overall system resilience.

By offering a dynamic platform for immediate electricity trading, spot markets are a critical product in regional power markets, supporting efficient operations, economic optimization, and the transition to sustainable energy systems.

Use cases Spot markets are integral to the operation and optimization of electricity grids across various regions. They enable efficient real-time trading, allowing market participants to balance supply and demand dynamically. These markets are widely used for cross-border electricity trading, where exchanges like Nord Pool, EPEX SPOT, and OMIE facilitate seamless integration of renewable energy and promote grid stability. In North America, systems such as PJM Interconnection, CAISO, and NYISO leverage spot markets for day-ahead and real-time balancing, ensuring efficient dispatch and renewable energy management. In Asia, platforms like the Indian Energy Exchange (IEX) and Japan Electric Power Exchange (JEPX) provide critical frameworks for short-term electricity trading, enhancing flexibility and market liquidity. Similarly, in Southern Africa, the Southern African Power Pool (SAPP) uses spot markets to strengthen regional electricity cooperation, while Australia’s National Electricity Market (NEM) utilizes them to maintain grid reliability and support renewable integration. These use cases demonstrate the versatility of spot markets in addressing the unique operational needs of diverse electricity systems worldwide.

### Day-Ahead

Day-ahead power trading is a fundamental component of electricity markets, enabling the buying and selling of electricity one day before its actual generation and delivery. This type of trading can occur either on the Day-Ahead Market (DAM) of a power exchange often referred to as a day-ahead auction or through Over-the-Counter (OTC) contracts between trading parties, such as power companies or large industrial consumers. In liberalized power markets, day-ahead trading is crucial for balancing electricity production and consumption portfolios. Power traders, utilities, and large consumers must ensure that their electricity supply and demand are balanced daily within what is known as a balancing group. The responsibility of maintaining this balance lies with the Balancing Responsible Parties (BRPs), who manage the portfolios to ensure that electricity bought/generated equals electricity sold/consumed.

Long-term balancing of these portfolios typically involves the use of futures markets, where BRPs hedge risks by purchasing or selling electricity for delivery months or years in advance. However, as the day of actual electricity delivery approaches, BRPs need to fine-tune their portfolios to address any remaining surpluses or deficits. The Day-Ahead Market facilitates this adjustment through competitive bidding and price discovery, providing a platform for short-term balancing of supply and demand. After the close of day-ahead trading, any residual imbalances in the balancing group can be corrected through intraday trading. This sequential approach, from futures to day-ahead and then intraday markets, ensures an efficient and reliable electricity market operation, minimizing risks and supporting grid stability. The day-ahead trading product thus plays a critical role in maintaining market efficiency, promoting transparency, and supporting the reliable operation of interconnected power systems. It serves as a vital tool for energy market participants to align their operations with real-time needs.

Use Cases Day-ahead markets have been widely adopted across global power exchanges, demonstrating their importance in electricity trading systems. In Europe, markets like Nord Pool and EPEX SPOT facilitate daily trading across multiple countries, supporting regional integration and efficient energy scheduling. The PJM Interconnection and NYISO in the United States use day-ahead markets for balancing supply and demand and ensuring market liquidity. In Asia, Japan's power market and the Indian Energy Exchange (IEX) rely on day-ahead trading to manage generation and consumption effectively. The Southern African Power Pool (SAPP) uses day-ahead markets to enable cross-border trading among member countries, while Australia's AEMO integrates this mechanism to balance its diverse energy mix. These markets play a pivotal role in coordinating electricity trading in liberalized and interconnected systems worldwide

### Ancillary Services/Products

Ancillary services are essential for the reliable and stable operation of power transmission and distribution systems and play a pivotal role in the setup of a regional power market. These services maintain grid stability by balancing supply and demand, ensuring voltage control, and providing flexibility to address contingencies or unexpected fluctuations. They are broadly categorized into frequency ancillary services, which stabilize grid frequency, and non-frequency ancillary services, which manage voltage levels and enable black-start capabilities for system recovery after outages. The increased deployment of Variable Renewable Energy (VRE) generation introduces variability and uncertainty into power system operations, making ancillary services even more critical. To address these challenges, TSOs typically deploy flexible on-demand resources, including generation, storage technologies like batteries and pumped hydro, and demand-side response measures. These solutions help maintain grid reliability and security while integrating renewable energy sources effectively. Ancillary services are thus indispensable in modern power markets, particularly for supporting regional markets where diverse energy mixes and cross-border interconnections require robust mechanisms to ensure seamless and stable operations.

In the context of a regional power market, ancillary services are particularly critical for harmonizing operations across multiple interconnected grids with varying demand-supply dynamics. Transmission system operators (TSOs) procure these services from sources such as conventional generation assets, storage technologies like pumped hydro and batteries, and reactive power control equipment like synchronous compensators and capacitor banks. These services enable seamless cross-border electricity flows, help manage grid imbalances caused by fluctuating renewable energy sources and ensure the resilience of the interconnected system. By providing a framework for stability and reliability, ancillary services support the efficient functioning of regional power markets. They facilitate optimal resource utilization across borders, enhance market liquidity, and enable the integration of renewable energy while reducing the need for costly contingency measures. Their inclusion in regional market designs is therefore indispensable for achieving operational efficiency and long-term energy security.

Key Features

- Regulation and Contingency Reserves: Power markets like those in the U.S. and Australia run energy auctions every five minutes, but grid balancing often requires faster responses, within seconds. To manage unexpected changes in generation or demand between market intervals, system operators procure standby services, such as "raise" or "lower" services in Australia, which are auctioned every five minutes alongside the main market. Generators can choose to bid for energy dispatch or standby services. Different reserves, like primary (responding within 15 seconds), secondary (30 seconds), and tertiary (15 minutes), cater to varying balancing needs, as seen in European markets. The rise of intermittent renewables like solar and wind, which lack inertia, increases the demand for fast-response reserves. Specialized submarkets for quick-response services, supported by hydro reservoirs and batteries, are becoming crucial to maintaining grid stability.

- System Stability: System operators are responsible for ensuring system stability, which is crucial for preventing grid collapse. They may rely on components like capacitors and inductors to control reactive power, which helps maintain voltage stability and prevent phase differences that could lead to failures. Operators may also call on generators to offset sudden outages or demand surges to avoid cascading collapses. As long as the system remains stable, electricity prices reflect the efficient dispatch of generation. However, during a collapse, there is no price for electricity. The provision of stability services is often competitively contracted, funded by levies on market participants, which may be implicit. System operators can direct generators to provide these services without direct payment, making it a cost of doing business, ultimately borne by end users.

- Procurement of Ancillary Services and Co-Optimization: In power markets, generators are compensated not only for energy or capacity but also for ancillary services. When evaluating investment decisions, it is essential for market participants to consider all payment streams, including those from ancillary services. These services can be procured through auctions, negotiated agreements, or independent purchases, such as strategic reserves held by system operators. While ancillary services are ultimately funded by consumers through levies, regulation and contingency services should ideally be procured in a way that allows for co-optimization with energy market decisions. For example, simultaneous auctions for standby capacity and energy, as practiced in Australia, help integrate ancillary services efficiently into the broader market, enhancing system reliability and operational efficiency in regional power markets.

New Ancillary Services/ Products

- Ramping products - The rise in Variable Renewable Energy (VRE) generation causes increased volatility in the net load curve, creating a need for fast ramping capabilities to maintain system stability. Conventional generation, typically used to manage these ramping requirements, may be displaced by VRE and is increasingly relied upon for backup power. When ramping is procured through energy markets without being identified as a separate ancillary service, it can lead to price distortions, as steep ramping requirements drive up market prices. To mitigate this, balancing markets can introduce specific ramping or flexibility products. For instance, the California Independent System Operator (CAISO) implemented Flexible Ramp Up and Flexible Ramp Down products to procure ramping capabilities in 5- and 15-minute intervals. These products allow any resource capable of providing the required ramping to participate, and compensation is based on the opportunity cost of not providing other services. This approach helps reduce price spikes associated with ramping shortfalls, increases the availability of fast ramping capacity, and ensures a more efficient balancing of the system.

- Fast frequency response provided by batteries - Grid frequency stability is essential for secure system operation, and sudden fluctuations in demand and supply can cause frequency deviations, threatening system reliability. Traditionally, conventional generators provided quick frequency response through autonomous governor control, but with the increasing share of VRE, this response may not suffice. Batteries, due to their rapid response times, are now increasingly used to provide fast frequency response and balancing services. As battery storage technology costs decrease, it is becoming more common for system operators to deploy batteries for frequency regulation. For example, National Grid in the UK created an ancillary service product to procure fast frequency reserves, contracting battery storage facilities for sub-second response services. In South Australia, the Hornsdale Power Reserve, a 100 MW/129 MWh lithium-ion battery, has significantly reduced the cost of frequency ancillary services. In Japan, large solar PV projects like the 38 MW Tomakomai project have incorporated battery storage to meet local frequency requirements. These developments highlight the growing role of batteries in ensuring grid stability while reducing costs associated with traditional frequency response mechanisms.

New market participants providing ancillary services

- Wind turbines providing inertial response - Inertial response traditionally comes from large thermal generators and hydropower plants, where synchronous generators adjust their speed to overcome frequency disturbances. This rapid response is a key element for maintaining grid stability. However, with the increasing integration of Variable Renewable Energy (VRE), such as wind and solar, these resources have generally been exempt from balancing responsibilities. Wind turbines, particularly those connected via power electronic converters, can provide "synthetic inertia" to help stabilize frequency during disturbances. During frequency surges, turbines can reduce generation, while during frequency drops, they can utilize their kinetic energy to increase output. This response is achieved by adjusting turbine blade angles or applying retarding torque. For instance, Hydro-Québec TransÉnergie in Canada required wind turbines to provide inertia emulation in its 2,000 MW wind energy procurement tender. The ability of wind turbines to deliver inertial response provides an opportunity to enhance grid stability, though it requires policy adjustments and system upgrades, such as specific grid codes and service procurement mechanisms, to fully leverage this potential

- Solar PV power plants and utility-scale storage providing reactive power - Reactive power is essential for maintaining voltage levels within prescribed limits in the power system. However, transmitting reactive power over long distances can lead to issues such as voltage fluctuations and increased losses, necessitating its provision from nearby sources. Traditionally, this has posed challenges for market mechanisms due to limited local sources. However, solar PV power plants and utility-scale storage, which connect to the grid via inverters, can offer reactive power support. This capability is particularly valuable in areas where renewable energy sources like solar and wind are located far from load centers, often linked to weak networks. To ensure these assets contribute effectively to reactive power control, appropriate grid codes and connection requirements must be in place. These measures may involve slight increases in capital expenditure for generators but can guide investment decisions. Additionally, separate procurement mechanisms for reactive power, though less common, could help incentivize its provision and enhance system stability.

- Distributed Energy Resources - Distributed Energy Resources (DERs), such as rooftop solar, behind-the-meter battery storage, plug-in electric vehicles, and commercial or industrial loads, can provide ancillary services to system operators through price-based incentives, also known as "explicit demand response." By increasing market liquidity and competition, DERs can help reduce ancillary service procurement costs. Depending on the market design, DERs may participate independently or through aggregators or retailers. For example, the New York Independent System Operator (NYISO) has proposed allowing DERs to participate in wholesale and ancillary service markets, including capacity reserve and regulation service markets. Additionally, DERs can engage in local flexibility markets, which help system operators address grid congestion and other issues. While DERs offer significant benefits, their integration into the distribution network may raise concerns regarding stability and reliability. As a result, decentralized management by Distribution System Operators (DSOs) is becoming more appealing, especially to address challenges such as voltage regulation, power quality, and network congestion caused by increased distributed generation.

Use Cases: Ancillary services are critical in ensuring the stability and reliability of power systems, especially as renewable energy sources like wind and solar increase their share in the energy mix. In the European Union, the European Network of Transmission System Operators for Electricity (ENTSO-E) enables the procurement of various ancillary services, such as primary, secondary, and tertiary reserves, through its integrated electricity market. These services are crucial for frequency control across interconnected grids, allowing for efficient renewable energy integration. In the United States, the California Independent System Operator (CAISO) introduced the Flexible Ramp Up and Flexible Ramp Down Uncertainty Awards in 2016, targeting fast response services from batteries due to rapid fluctuations in net load caused by variable renewables. Similarly, Australia’s National Electricity Market (NEM) runs auctions for regulation and contingency reserves, where power plants bid for standby services, enhancing system flexibility and enabling better integration of renewables. The United Kingdom has implemented the Enhanced Frequency Response (EFR) product, allowing battery storage systems to provide sub-second response services for frequency regulation, significantly lowering service costs. In Japan, projects like the Tomakomai Solar PV system with battery storage contribute to grid stability by providing reactive power and frequency regulation. Meanwhile, in New York, the New York Independent System Operator (NYISO) has developed a market framework to allow Distributed Energy Resources (DERs) such as rooftop solar, battery storage, and demand response to participate in ancillary service markets, increasing competition and lowering procurement costs. Lastly, in the Nordic electricity market, hydropower plants and wind turbines play a key role in offering frequency and voltage regulation services, with wind turbines providing synthetic inertia to maintain system stability.

### Derivatives

Derivative trading in the energy market is a vital tool for managing price volatility, which can result from factors such as weather conditions, fuel price fluctuations, system outages, and variability in renewable energy generation. Electricity derivatives, financial instruments tied to underlying electricity prices, enable market participants to hedge against these risks. These instruments not only provide a means to mitigate the effects of price instability but also offer opportunities for both risk management and speculative trading. The European Energy Exchange (EEX) exemplifies a model where electricity derivatives are traded alongside physical electricity, allowing participants to optimize their exposure to price fluctuations. In contrast, markets like the Intercontinental Exchange (ICE) and Singapore Exchange (SGX) operate separate platforms for electricity and derivatives, each providing distinct advantages for price discovery and risk management. These market structures enhance the overall efficiency and stability of global electricity markets. A derivatives exchange focuses on the future delivery of electricity, with companies using it to hedge against price-related risks. Contracts typically involve an agreed-upon amount of electricity at a set price for a specific period, ranging from months to years, without the physical transfer of electricity, but only through financial settlements. The NASDAQ Commodities exchange is the leading derivatives exchange in the Nordic region.

Working Principles

An electricity derivatives market enables consumers and producers to manage future electricity price risks by locking in prices for upcoming months or years. The market operates as a continuous auction, where buyers and sellers meet to establish the market price through bids and offers. This process ensures price transparency and helps participants gauge the future price level. One of the main challenges of derivative trading is the financial commitment involved. To participate, companies must have sufficient capital (cash or collateral) from the outset and the capacity to handle daily reporting and financial transactions. As a result, exchange members typically include major companies, banks, and firms specializing in exchange trading.

Exchanges are designed to minimize risks and facilitate efficient transactions. They address counterparty risks by ensuring that all participants can fulfill their obligations. This is achieved through requirements for collateral or guarantees from participants. To begin trading, participants must deposit collateral into a clearing account, with additional collateral added daily to meet margin requirements based on ongoing transaction obligations.

Collateral Requirements

Collateral requirements are essential components of derivative transactions, consisting of various elements such as initial margin and variation margin. These requirements fluctuate in response to market price changes, meaning that even without entering into new derivatives contracts, shifts in market prices can lead to an increase or decrease in the collateral needed. For instance, if a seller has committed to a contract at a low price and market prices rise, the seller must deposit additional collateral with the exchange. Conversely, if prices fall, the collateral is returned. Once the product is delivered, the collateral associated with that specific trade is fully refunded.

The primary purpose of collateral is to ensure that other participants in the market are compensated in the event that a market participant fails to meet its obligations—such as in cases of default or bankruptcy. Recent spikes in energy commodity prices, particularly from late 2021 onward, have led to a substantial increase in collateral requirements for exchange trading. Electricity market participants, including sellers and buyers, have been securing future production and consumption contracts, which were in place even before the price surge. As a result, companies are required to deposit collateral to reflect the current price levels, increasing the financial burden. The escalation of collateral requirements has resulted in higher indirect costs and liquidity challenges for companies, as significant amounts of capital are tied up in collateral, impacting overall financial flexibility.

Types of Products in Derivatives Market

- Forwards (OTC, Physical or Financial) - This involves the agreement to buy or sell electricity at a pre-specified contract price for delivery at a future date. The buyer is obligated to take delivery of a fixed amount of electricity, and the seller is obligated to supply it at the agreed forward price, at the specified maturity or expiration time. These contracts are typically customized supply agreements between a buyer and a seller. Electricity forward prices are determined based on long-term expectations and typically exhibit stable behavior. Long-term forwards generally experience lower volatility, whereas short-term forwards may face higher volatility. These prices are often correlated with fuel prices.

- Futures (Exchange, Financial) - Electricity futures contracts are primarily settled through financial payments, or cash settlement, which lowers transaction costs compared to physical delivery. These contracts are highly standardized, with fixed contract specifications, defined trading locations, specific transaction requirements, and standardized settlement procedures. The key difference between futures and forwards is the quantity of power to be delivered, with futures contracts typically specifying smaller delivery quantities than forward contracts. The advantages of electricity futures include market consensus, price transparency, and trading liquidity, which help reduce transaction and monitoring costs. However, the main disadvantage is the lack of customization, as only standardized contracts can be traded, offering less flexibility compared to forward contracts.

- Swaps or CfD (OTC, Financial) - Swaps, or Contracts for Difference (CfD), are financial contracts where the holder exchanges a fixed price for electricity against the market's variable price over a contracted period. These contracts are settled based on the market price, with the fixed cash flow exchanged for a variable cash flow stream. Essentially, the holder pays a fixed price for electricity, regardless of the fluctuating market price, or vice versa, depending on the terms of the contract. Swaps are typically established for a fixed quantity of power, referenced to a variable spot price, which may be linked to either a generator's or a consumer's location. These contracts are commonly used for short- to medium-term price certainty, providing stability for periods up to a couple of years. A swap can be seen as a strip of electricity forwards with multiple settlement dates, each having an identical forward price, providing flexibility in managing price risks across multiple periods.

- Options (Exchange, Physical or Financial) - Options provide buyers with the right, but not the obligation, to buy or sell electricity at a predetermined price, in exchange for a premium paid upfront. The buyer has the flexibility to choose whether to exercise the option, while the seller is obligated to deliver or take the asset if the option is exercised. This structure is similar to insurance, where the buyer pays an annual premium, and the seller compensates for any potential damages. There are two main types of options: Call options, which give the holder the right to buy electricity at a specified strike price, and Put options, which grant the right to sell at a predetermined price. The strike price is the price at which the underlying commodity (electricity) can be bought or sold. The value of the option contract is determined by how close the market price is to the strike price, influencing whether the option holder exercises the right to buy or sell.

Examples of above mentioned products are mentioned in Appendix B.

Use Cases: Derivatives markets play a pivotal role in electricity trading by allowing market participants to manage risks associated with price volatility and market uncertainties. For example, the European Energy Exchange (EEX) offers a model where electricity derivatives, such as Futures and Options, are traded alongside physical electricity. This integrated platform allows traders to optimize their exposure to price fluctuations, with electricity futures and options providing the tools necessary for hedging against the volatility caused by factors like demand-supply imbalances, weather variations, and fuel price changes. This model is particularly effective for entities seeking both price discovery and risk management, as participants can trade both financial products and physical electricity within the same exchange. On the other hand, exchanges like the NASDAQ OMX Commodities Exchange, NYMEX, and ICE Futures US specialize in Futures contracts, enabling participants to buy or sell electricity at a fixed price for future delivery. These exchanges, primarily dealing in Futures, provide a structured environment for hedging against price volatility, allowing companies and traders to lock in prices in advance and stabilize their operations. For instance, companies in the United States utilize NYMEX’s electricity futures contracts to hedge against regional price fluctuations, while participants in Europe use EEX products to navigate broader market volatility. Meanwhile, platforms like the Intercontinental Exchange (ICE) and the Singapore Exchange (SGX) separate electricity and derivatives trading onto distinct platforms. This separation enhances price discovery and risk management by allowing participants to focus specifically on either the physical or the financial aspects of the market. For example, ICE Futures US provides futures contracts for electricity trading, while SGX offers electricity futures to manage risks in the Asia-Pacific region.

From an institutional standpoint, the establishment of a Regional Power Exchange, offering key market products such as spot, day-ahead markets, ancillary services, and derivatives, is crucial for ensuring market efficiency, price stability, and risk management. These market products not only facilitate seamless trading but also support the integration of renewable energy, improve system reliability, and ensure the flexibility needed to address unforeseen market fluctuations. As already mentioned earlier, the integration of regional power markets requires a comprehensive approach that addresses the political, technical, and institutional prerequisites for effective cooperation. Politically, alignment of national policies and a commitment to cross-border collaboration are vital to create a unified framework for market operation. Technically, the development of a robust grid infrastructure, including interconnectors and advanced systems for balancing supply and demand, is essential to ensure efficient market functioning. By addressing these foundational requirements, regional power markets can become more resilient, efficient, and sustainable, ultimately fostering greater energy security and economic cooperation across borders.

# Appendix A: CBET Transmission Line Details

Details of cross-border transmission lines in BBINS region:

A. Interconnection between India and Nepal

- Existing/ operational lines:

  - 400 kV D/C: Dhalkebar – Muzaffarpur Line

  - 220 kV: Hetauda – Bharatpur Transmission Line (operated at 132 kV)

  - 132 kV: Mainahiya – Sampatiya Line

  - 132 kV D/C: Kusaha – Kataiya Line

  - 132 kV: Kusaha – Kataiya Line (Second Circuit)

  - 132 kV S/C: Ramnagar – Gandak (Surajpura) Line

  - 132 kV S/C: Mahendranagar – Tanakpur Line

  - 132 kV S/C: Raxaul – Parwanipur Line

  - 132 kV: Raxaul – Parwanipur (Second Circuit) Line

  - 33 kV S/C: Birganj – Raxaul Line

  - 33 kV S/C: Jaleshwar – Sitamarhi Line

  - 33 kV S/C: Sirha – Jaynagar Line

  - 33 kV S/C: Rajbiraj – Kataiya Line

  - 33 kV S/C: Biratnagar – Kataiya Line

  - 33 kV S/C: Nepalgunj – Nanpara Line

  - 33 kV S/C: Mahendranagar – Lohia Line

  - 11 kV: Baitadi – Pithoragarh Line

  - 11 kV: Jaljibe – Dharchula Line

  - 11 kV: Pipli – Dharchula Line

- Proposed/ underconstruction lines:

  - 400 kV D/C: Butwal – Gorakhpur Line

  - 400 kV D/C: Muzaffarpur – Dhalkebar (Second Line)

  - 400 kV D/C: Lumki – Bareilly Line

  - 400 kV D/C: New Duhabi – New Purnea Line

  - 400 kV D/C: Sitamarhi – Dhalkebar Line

  - 400 kV D/C: Hetauda – Dhalkebar – Inaruwa Line

  - 132 kV: Nepalgunj – Nanpara Cross-Border Line

B. Interconnection between India and Bangladesh

- Existing/ operational lines:

  - 400 kV D/C: Baharampur – Bheramara Line

  - 400 kV (operated at 132 kV): Surjyamaninagar – North Comila – South Comila Line

  - 400 kV Godda Bogura D/C transmission line

- Proposed/ underconstruction lines:

  - 765 kV D/C Katihar – Parbotipur – Bornagar Line

B. Interconnection between India and Bhutan

- Existing/ operational lines:

  - 400 kV Binaguri(ER,IN) – Tala I & II(BH)

  - 400 kV Binaguri(ER,IN)– Tala IV (BH)

  - 400 kV Binaguri(ER,IN) –Malbase(BH)

  - 400 kV Alipurduar (ER,IN)–Jigmeling (through Punatsangchhu)(BH)

  - 220 kV Birpara(ER,IN) –Chukha(BH)

  - 220 kV Birpara(ER,IN) –Malbase(BH)

  - 132 kV Salakati(NER,IN) –Gelephu(BH)

  - 132 kV Rangia(NER,IN) –Motonga(BH)

- Feeder connection (between Bhutan and Indian DISCOM):

  - 33 kV Tamalpur - Sandrupkhonar (Assam-NER grid)

  - 11 kV Rauta - Diafem (Assam-NER grid)

D. Interconnection between India and Sri Lanka

Proposed line: 1000 MW HVDC bipole line connecting New Madurai in India to Mannar in Sri Lanka (DPR stage completed)

Details of Interconnection between India and Bangladesh

| Sl no. | Interconnection Project | Voltage and Configuration | Intercon. Type |  | Evacuation Capacity (MW) | Traded Capacity (MW) | Interconn. Point in India | Interconn. Point in Nepal | Tentative Comm Date | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status – Existing Projects |  |  |  |  |  |  |  |  |  |  |
|  | Dhalkebar – Muzaffarpur Line | 400 kV D/C | AC |  | 1000 | 400-500 | Bihar | Dhanusa District |  | Owned by a JV company of PGCIL, SJVN and NEA on Indian side and JV of NEA/PGCIL/HIDCL/IEDCL |
|  | Hetauda-Bharatpur Transmission Line | 220 kV | AC |  |  |  | Rajasthan | Makwanpur district | 2023 | Currently charged at 132 kV |
|  | Mainahiya – Sampatiya Line | 132 kV | AC |  | 100 |  | Uttar Pradesh | Rupandehi District |  | completed in 2023 |
|  | Kusaha - Kataiya Line | 132 kV D/C | AC |  | 130 | 80 | Bihar | Duhabi |  | On the Indian side owned by the respective state transmission utilities/distribution utilities On the Nepal side owned by NEA |
|  | Kushaha – Kataiya Line (Second Circuit) | 132kV | AC |  | 100 |  | Bihar | Duhabi |  |  |
|  | Ramnagar- Gandak (Surajpura) | 132 kV S/C | AC |  | 50 | 25 | Bihar | Surajpura |  |  |
|  | Mahendranagar – Tanakpur | 132 kV S/C | AC |  | 80 | 40-70 | Uttarakhand | Mahendranagar |  |  |
|  | Raxual -Parwanipur | 132 kV S/C | AC |  | 70 | 50 | Bihar | Bara District |  |  |
|  | Raxual – Parwanipur (Second Circuit) | 132 kV | AC |  | 100 |  | Bihar | Bara District | Charged on January 2024 |  |
|  | Birganj – Raxaul | 33 kV S/C | AC |  | 10 | 10 | Bihar | Sudurpaschim Province |  |  |
|  | Jaleshwar - Sitamarhi | 33 kV S/C | AC |  | 10 | 8 | Bihar | Janakpur Zone |  |  |
|  | Sirha – Jaynagar Line | 33 kV S/C | AC |  | 8 | 8 | Bihar | Province No. 2 |  |  |
|  | Rajbiraj – Kataiya Line | 33 kV S/C | AC |  | 10 | 8 | Bihar | Saptari District |  |  |
|  | Biratnagar – Kataiya Line | 33 kV S/C | AC |  |  |  | Bihar | Rupri |  |  |
|  | Nepalgunj – Nanpara Line | 33 kV | AC |  | 10 | 8 | Uttar Pradesh | Banke |  |  |
|  | Mahendranagar – Lohia Line | 33 kV | AC |  |  |  | Uttarakhand | Mahendranagar |  |  |
|  | Baitadi – Pithoragarh Line | 11 kV | AC |  |  |  | Uttarakhand | Sudurpashchim Province |  |  |
|  | Jaljibe – Dharchula Line | 11 kV | AC |  |  |  | Uttarakhand | Jaljibe |  |  |
|  | Pipli – Dharchula Line | 11 kV | AC |  |  |  | Uttarakhand | Pipli |  |  |
|  | Butwal Gorakhpur Line | 400 kV D/C | AC |  | 1000 | - | Bihar | Lumbini Province | 2025 | Proposed to be developed by a JV of NEA and PGCIL |
|  | Muzaffarpur-Dhalkebar -2nd Line | 400 kV D/C | AC |  | 1000 | - | Bihar | Dhalkebar | >2025 | Associated with Arun 3 project. |
|  | Lumki-Bareilly | 400 kV D/C | AC |  | 1000 | - | Bareilly | Lumki | >2025 |  |
|  | New Duhabi-New Purnea | 400 kV D/C | AC |  | 1000 | - | Bihar | New Duhabi | >2030 |  |
|  | Sitamarhi – Dhalkebar Line | 400 kV D/C | AC |  | 1000 |  | Bihar | Dhanusa District |  |  |
|  | Hetauda-Dhalkebar-Inaruwa | 400 kV D/C | AC |  | 2000 |  |  | Hetauda- Dhalkebar-Inaruwa | Mid-2025 |  |
|  | Nepalgunj-Nanpara Cross Border | 132 kV | AC |  |  |  | UP | Banke District |  |  |

Interconnection between India and Bangladesh

| Sno. | Interconnection Project | Voltage and Configuration | Intercn. Type | Evac. Capacity (MW) | Traded Capacity (MW) | Intercn. Point in India | Intercn. Point in Bangladesh | Tentative Commissioning Timelines | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status – Existing Projects |  |  |  |  |  |  |  |  |  |
|  | Baharampur – Bheramara Line | 400 kV D/C | HVDC | 1000 | 1000 | West Bengal (Baharampur) | Bheramara |  | On the Indian side owned by PGCIL, On the Bangladesh side owned by PGCB |
|  | Surjyamaninagar – North Comila – South Comila | 400 kV (operated at 132 kV) | HVAC | 160 | 160 | Tripura (Suraj Maninagar) | North Commila |  |  |
| Status – Proposed Projects |  |  |  |  |  |  |  |  |  |
| 3. | Katihar – Parbotipur - Bornagar | 765 kV D/C | HVDC | 1000 MW |  | - Bihar (Katihar) - Assam (Bornagar) | Parbotipur | >2026 | Under discussion |

# Appendix B: Examples of Derivative Trading

i. Forward:

- Forward Contract Entered in Jan’25 for Delivery in June ‘25

- Two types of Credit Risk

  - Replacement Risk:- Before Start of Delivery if any counterparty defaults. For ex. If

Buyer (B) defaults on March 17 to take power from Seller (S), then Seller has to enter in a new contract at current market price, which will be generally at low price say @Rs. 3/kWh with a new counterparty. So Replacement Risk=(4-3)*Contract Volume

  - Settlement Risk:- If the electricity is delivered but buyer defaults to make payment,

This creates settlement risk which has generally several times higher risk than replacement risk=4*Contract Volume. In addition to this delay in payment also comes in settlement

risk

Credit risk exposure is defined as the sum of the settlement and the replacement risk.

ii. Futures:

- Generator hedges 100 MW load in Futures Market

- Sells Futures Contract at a future price in Jan ‘25 @ Rs 4/kWh which settles at spot market price

Scenario 1:- Avg. spot market price during delivery period say Rs 3/kWh

Scenario 2:- Avg. spot market price during delivery period say Rs 5/kWh

iii. Swaps:

- Bank agrees to pay Generator Rs 3500/MWh for 50 MW of power during December 2025.

- Generator agrees to pay Bank cash flows equal to a floating price on the same quantity of electricity for one year.

- By combining this swap with the indexed electricity supply contract, a Generator can lock in a fixed income and sell to PX

iv. Options:

- Generator hedges 100 MW load in Options Market

- Buy a put contract in Jan ‘25 at a strike price of Rs 4/kWh by paying a premium @ Rs. 0.5/kWhr

Scenario 1:- Avg. spot market price during delivery period Rs 3/kWh. Hence Exercise Option

Scenario 2:- Avg. spot market price during delivery period Rs 6/kWh, Option not exercised

Source: BPDB Annual Report 2022-23 (https://bpdb.gov.bd/site/page/c4161d54-5b85-4917-a8d2-68a2d1b26dd4/Monthly-Annual-Report)

Source: Bhutan Energy Data Directory 2022 (https://www.moenr.gov.bt/wp-content/uploads/2018/11/Final-copy-of-BEED-2022.pdf)

NEA Annual report 2023/2024 (https://www.nea.org.np/admin/assets/uploads/supportive_docs/60896609.pdf)

Source: Annual Performance Report 2023 (https://www.parliament.lk/uploads/documents/paperspresented/1720521475057956.pdf)

Source: Bangladesh Bank Annual Reports (https://www.bb.org.bd/en/index.php/publication/publictn/0/2)

Source: Report on Asian Development Outlook, September 2024 (https://www.adb.org/sites/default/files/publication/995536/asian-development-outlook-september-2024.pdf)

Source: Bangladesh Bank Annual Reports (https://www.bb.org.bd/en/index.php/publication/publictn/0/2)

Source: https://www.nsb.gov.bt/publications/statistical-yearbook/

CEA Monthly Power Supply Reports

Ministry of Power and Energy- Annual Performance Report 2023 (https://www.parliament.lk/uploads/documents/paperspresented/1720521475057956.pdf)

Source: BPDB Annual report 2022-23

SAREP report on Coordination of Policies, Legal and Regulatory Frameworks for Cross Border Electricity Trade of Bangladesh (https://sarepenergy.net/wp-content/uploads/2023/11/Coordination-of-Policies-Legal-Regulatory-Frameworks-for-CBET-of-Bangladesh-by-S.K.-Munir-Ahmed-Director-Power-Cell-MPEMR.pdf)

CERC (https://cercind.gov.in/2023/market_monitoring/Annual-2022-23/Chapter-III/AR-22-23_Chapter-III.pdf)

SAREP report (https://sarepenergy.net/wp-content/uploads/2023/11/Sri-Lanka-Updates-on-Coordination-of-Policies-Legal-and-Regulatory-Framework-by-Mr.-Sugath-Dharmakeerthi-Additional-Secertary-MoP.pdf)

Source: http://nvvn.co.in/wp-content/uploads/2020/11/Nomination-of-NVVN-as-SNA.pdf

SAARC member states are Afghanistan, Bangladesh, Bhutan, India, the Maldives, Nepal, Pakistan and Sri Lanka.

BIMSTEC member states are Bangladesh, Bhutan, India, Myanmar, Nepal, Sri Lanka, and Thailand.

Source: IEA report on multilateral power trading (https://www.iea.org/reports/establishing-multilateral-power-trade-in-asean)

Source: IEA report on multilateral models (https://www.iea.org/reports/establishing-multilateral-power-trade-in-asean)

Source: Report on Establishing multilateral power trade in ASEAN (https://www.iea.org/reports/establishing-multilateral-power-trade-in-asean)

Source: SAREP report on transition to trilateral and multilateral power trade in South Asia (https://sarepenergy.net/work/study-on-the-transition-to-trilateral-and-multilateral-power-trade-in-south-asia-irade-sari-37-2021/)

Source: USAID/ IRADe report (https://irade.org/Transition%20to%20Trilateral%20&%20Multilateral%20Trade_Final%20Report%20Print%20Reday%20Version.pdf)

Source: Pentalateral Energy Forum: Generation Adequacy Assessment Report 2020 (https://www.swissgrid.ch/dam/swissgrid/about-us/newsroom/newsfeed/2020/2020-PLEF-GAA-3-0-report.pdf)

Source: IEA (https://www.iea.org/reports/institutional-architecture-for-regional-power-system-integration)

Source: IEA report on Institutional Architecture for Regional Power System Integration (https://www.iea.org/reports/institutional-architecture-for-regional-power-system-integration)

Source: SAREP Website (https://sarepenergy.net/events/workshop-on-international-best-practice-on-business-and-financial-models-for-developing-cross-border-electricity-transmission-infrastructure/)

Source: IEA (https://www.iea.org/reports/establishing-multilateral-power-trade-in-asean)

Source: SAREP (https://sarepenergy.net/wp-content/uploads/2022/07/Presentation_on_Draft_Interim_Findings_of_the_TF-1_Study.pdf)

Source: Nord Pool (https://www.nordpoolgroup.com/en/message-center-container/newsroom/exchange-message-list/2024/q1/nord-pool-reports-robust-trading-figures-for-2023/)

Source: European Union (https://energy.ec.europa.eu/topics/infrastructure/electricity-interconnection-targets_en)

The Nordic Council of Ministers is the official inter-governmental body for cooperation in the Nordic Region.

Source: IRENA (https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2019/Feb/IRENA_Innovative_ancillary_services_2019.pdf?la=en&hash=F3D83E86922DEED7AA3DE3091F3E49460C9EC1A0)

Source: NEA Annual Reports, ADB Reports

Source: ORF Reports, ADB Reports, MoP (https://powermin.gov.in/en/content/interconnection-neighbouring-countries#:~:text=A%20high%20capacity%20interconnection%20between,India%20to%20Comilla%20in%20Bangladesh.)

Source: IRADE report on CBET (https://irade.org/Final%20report%20of%20study_NSB%20India.pdf)