Geographic Information System (GIS) – Power Utilities in India

Background

GIS is a digital representation of the elements of a electrical network (Transmission/Distribution) including customers, assets on a geographic map with appropriate resolution and attributes. It is a system which captures, stores, analyses, manages, and presents data (like sub-stations, feeders, transformers, poles, meters, cables & consumers) linked to the location(s).

Benefits of GIS

- Involves mapping of all network assets (substations, feeders, transformers and poles), wherein asset repository is built, which when integrated with ERP software, facilitates integrated asset management,

- With the asset repository depicted in a graphical, geo-referenced manner, fault location, predictive maintenance capabilities can be leveraged

- Consumers can be mapped to respective network assets, which can help in load planning, forecasting and management, improving response to consumer complaints etc.

- Consumer indexing enables the utility in understanding details of consumers such as category, connected load, meter details which helps the utility in identification of loss clusters.

- Work management can be improved with respect to route optimization, scheduling maintenance calls, network augmentation etc.

- Analytics can be carried out using network and consumer attribute data to help in reducing losses and optimizing costs.

Inception of GIS in Power Utilities in India

The National Electricity Policy 2005 identified the need for adopting modern information technology systems for creation of network information, customer data base, which will help in management of load, improvement in quality, detection of theft and tampering, customer information and prompt and correct billing and collection. The policy recognized the need for carrying out consumer indexing and mapping, in a time bound manner. Thus, policymakers recognised the importance of GIS, which was then given a further impetus through the Restructured Accelerated Power Development and Reform Program (R-APDRP) scheme of the Government of India (GoI). Part-A of the R-APDRP scheme involved preparation of Base-Line Data for the project area covering Consumer Indexing, GIS Mapping, Metering of distribution transformers and feeders and Automatic Data Logging for all distribution transformers and feeders and SCADA/DMS system. It includes Asset Mapping of the entire distribution network at and below the 11 KV Transformers and include the distribution transformers and feeders, L.T lines, poles and other distribution Network equipment.

Most of the power distribution utilities in India have implemented GIS, with private utilities being successful in leveraging its capabilities to the fullest, while State Government owned utilities have faced challenges, issues in the implementation and maintenance of GIS technology. Some of the challenges, issues encountered by State Government Utilities are outlined briefly herein below.

Challenges faced in GIS implementation by State Government Utilities in the Power Sector in India

Non-maintenance of data records: Utilities do not maintain records and if they do, they are not in digital format in many instances. This leads to dependence on field staff, which becomes a challenge, due to their lack of bandwidth, and/or capability.

Non-updation of GIS: One major challenge in GIS implementation, and which has largely contributed to low adoption of GIS is that subsequent changes in electrical assets and consumers are not updated in GIS, which defeats the very purpose of GIS including assessment of network capacity vis-à-vis additions required, identification of assets to be utilized for providing connections etc.

Lack of Capacity: Enterprise GIS technology and its integration with other systems is complex and the utilities have not been able to build internal capacity to handle such complex implementations, which combined with lack of Vendor support, has led to stalling in adoption of GIS.

High Cost of GIS Software: Many of the utilities have implemented best-in-class GIS software, with custom development done, which is not cost-effective from a long-term maintenance perspective.

Possible Solutions

GIS involves significant expenditure in terms of software, hardware, infrastructure and involvement of system integrator. Therefore, utilities need skilled and adequate resources to ensure timely and complete implementation and smooth post-go-live operations. However, as utilities lack these resources, and this is one of the key reasons for failures/issues in GIS implementations across utilities in India. Recently, Utilities have come out with tenders for GIS implementation, however, it is seen that some of issues are likely to continue in the future as well.

In this regard, some of the issues and our recommendations on the same are given herein below for kind consideration:

| Scope | Position in current GIS Tenders | Recommendations |
| --- | --- | --- |
| GIS implementation | Some of the tenders issued include development/implementation of GIS software, carrying out consumer indexing and providing maintenance support is bundled with network modeling | Separate tenders should be issued for implementation of GIS software & its maintenance, consumer indexing and network modeling, as these three requirements involve different skillsets and resource requirements, which may not be available with a single firm. |
| Project Management Support | Currently, State Government Utilities have not considered any project management support | Considering the large expenditure, long gestation period and complexity, it is recommended that project management support from agencies possessing relevant capabilities should be onboarded by State Government Utilities seeking to implement GIS technologies. |
| Vendor Selection method | GIS technology is complex, time-consuming to implement and need adequate resources for successful implementation and maintenance, therefore vendor selection should be driven by quality requirements rather than only cost. | It is recommended that QCBS method should be adopted for selection of vendors for the different requirements for GIS implementation and maintenance |
| Easy-to-Use and Implement solutions | Currently, utilities are focusing on implementing best-of-breed solutions for GIS. However, it is seen that these solutions are complex, costly to implement and maintain. Utilities do not have adequate and skilled resources for maintenance of GIS, which leads to non-realization of the benefits. | It is recommended that easy-to-use solutions based on open-source technology, which are much cheaper and provide similar capabilities should be encouraged to be implemented. For this, the eligibility and evaluation criteria should be relaxed to accommodate such solutions. |

Source: Performance Audit Report on Centralised Information Technology Billing System being operated by State Power Utilities in Uttar Pradesh issued by CAG India